- ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

GENERAL PART - The Contractor is responsible for advising of any H&S risks associated with equipment provided and of suitable protective measures. - All computers, servers, printers and ancillary equipment shall be delivered, installed, and be fully operational at the corresponding site locations. - All equipment must be oferred from the manufacturere’s bussines line (home and SoHo will not be allowed) and must be supplied with full technical documentation comprising of at least, operation manual and service manual. This documentation must be supplied in 2 copies in English language. - The Contractor shall provide or secure the provision of a local reliable after-sales service thereafter guaranteeing maintenance and the rapid replenishment of spare parts and consumables for at least 2 years (see Art. 33 of SC). His technical offer shall include a description of how the service will be provided by a local organization authorized by the manufacturer including name of the service organisation, postal address, telephone and facsimile number and e-mail address. The after-sales service provided for, shall include but not be limited to the following: - A telephone “Hotline” for rendering advice on equipment use. The “hotline” shall be available during working hours, 8am to 6pm. - Possibility to submit technical support requests by fax and e-mail with a maximum response time 1 working day. - Confirm that any requests for services will be attended to within 24 hours. - Confirm that all items can be repaired or alternatively replaced within a maximum of 72 hours. - Confirm that genuine spare parts and consumables will be available for a period of minimum 5 years from the date of final acceptance of the equipment. - Preparation of tender specifications: - Tenderers are to offer standard production models at least matching or exceeding the specifications, stated in the outline specifications – see detailed Specifications - Platform stability: 18-months platform stability - Tenderer shall clearly state in the attached lists the proposed manufacturers, the models and the country of origin. Manufacturer's technical literature and leaflets must be provided as part of the Technical Proposal. 6. Please note that in attached tables: - Columns 2 is completed by the Contracting Authority and shows the required minimum specifications (not to be modified by the tenderer), - Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient) - Column 4 allows the tenderer to make comments on his proposed supply and to make eventual references to the documentation The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee. The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

- ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Instructions:

- Columns 1-2 are completed by the Contracting Authority

- Columns 3-4 should be completed by the tenderer

- Column 5 is reserved for the evaluation committee

Contract title: Supply of Video Conferencing-Audiovisual and Computing Equipment with peripherals for Distance Learning via the Serbian School of Health Management, to be used by four (4) Medical Faculties in the Republic of Serbia

Publication reference: EuropeAid/127373/D/SUP/RS. 							Page No […of…]

| LOT 1 : Video Conferencing-Audiovisual Equipment with peripherals for Distance Learning |  |  |  |  |
| --- | --- | --- | --- | --- |
| Item Number | Specifications | Specifications Offered | Notes, remarks, ref to documentation | Evaluation Committee’s notes |
| 1.1 | Plasma 50” TV (Minimum Specification) State Manufacturer: State Model: State Country of Origin: Display Type: Plasma Diagonal size: Minimum 50” Audio/Video Input Signal: Composite ; S-Video, Component Audio RCA L+R; TV In/Coax; HDMI TV Tuner: Tuner bands : Hyper band, S channel, UHF, VHF TV system : PAL, SECAM Video playback : NTSC, PAL, SECAM Native Resolution: Minimum 1365 x 768 Aspect Ratio: 16:9 Digital TV Capability: 480i, 480p, 720p, and 1080i Brightness (typical): Minimum 450 cd/m2 Contrast Ratio: Minimum 10000:1 Horizontal Viewing Angle: +/- 80 degrees Vertical Viewing Angle: +/- 80 degrees Audio output power: 20W POWER CHARACTERISTICS: Voltage 220-240V Frequency 50-60Hz Power cord appropriate for Serbian electrical sockets |  |  |  |
| 1.2 | SLR Digital Still Camera + Zoom lenses 18-55 & 55-200 (Minimum Specification) State Manufacturer: State Model: State Country of Origin: Sensor type :CMOS # Imaging Device: 10,1 Megapixel (minimum) # A/D Conversion 14bit # ISO: Auto, 100, 200, 400, 800, 1600 (minimum) # File formats : RAW, JPEG Dust reduction Auto focus: 9 point Focus modes: One shot AF, AI Servo AF, AI Focus AF, Manual focus AF point selection, Auto, Manual AF assist light Metering modes: Spot metering, Center-weighted Exposure compensation AE lock Shutter: 1/80000 Viewfinder: Eye-level pentaprism LCD monitor: 3.0 " TFT LCD (minimum) Internal Flash: Auto pop-up # External flash:, hot-shoe connector # Zoom Lenses: 18-55 f/3.5-5.6 High-speed Auto-focus Optimised lens coatings Lead-free glass optical system # Zoom Lenses: 55-200, f/4.5-5.6 Compact and lightweight telephoto zoom lens High-speed Auto-focus Optimised lens coatings Lead-free glass optical system # 2GB MB Memory card min. CF,SD, MMC, USB 2.0 Card Reader # Supplied Software for: Microsoft® Windows® Professional/XP Home/Pro & MS Windows Vista |  |  |  |
| 1.3 | HDV Camcoder (Minimum Specs) State Manufacturer: State Model: State Country of Origin: Signal System: DV/HDV: 1080/50(60)i, 1080/25p Image Device: 1/3" 3CMOS Sensor system Lens: HD Video Lens supplied Zoom Lens, 12x (Optical) 35mm eqv: 38.9-380 mm) filter diameter: 72mm Horizontal Resolution: 1080 Lines LCD Monitor: 3.0", 16:9 Aspect Ratio, 1 MegaPixel Xtrafine LCD Shutter Speed Range: Auto: 1/60-1/2000, Manual 1/3 - 1/10000 VTR HDV Record and Play DV Record and Play Audio Audio Signal Format: HDV MPEG1 Audio Layer II DV 16-bit/48kHz 2 Channel Mode 12-bit/32kHz 4 Channel Mode Input and Output Connectors: ## HDV/DV terminals: Input/Output ## FireWire/i.LINK Headphone: Stereo Mini-Jack Microphone inputs component jack HDMI Transport bag Tripod Additional external microfone+cable 1x spare battery pack Driver Software for MS Windows XP, MS Windows Vista |  |  |  |
| 1.4 | Digital Audio Recorder (Minimum Specs) State Manufacturer: State Model: State Country of Origin: Type: Hand-Held Stereo audio SD Card Recorder Recording/Reading Methods: SD Card and USB Connection to PC/Mac Recording Bit Rate: WAV - Up to 24-bit, MP3 - Up to 320kbps Sampling Frequency 22.05/44.1/48/96kHz Frequency Response 20Hz - 40kHz or better Microphones: Two onboard Inputs: 2 x XLR - 1/4" Phone Combo Jacks (Balanced, Unbalanced), 2 x Built-In Condenser Microphones, 1 x USB Port Output: 1 x Master 1/8" (3.5mm) Mini Stereo Phone Jack. 1 x USB Port Headphone Output: 1 x 1/8" (3.5mm) Mini Stereo Phone Jack LCD Displey SD card 2GB External microphones: 2 omni-directional microphones with tripods |  |  |  |
| 1.5 | DLP Projector (Minimum Specification) State Manufacturer: State Model: State Country of Origin: System: PAL, SECAM, PAL-M, PAL-N Native Resolution (Minimum): True XGA 1024 x 768 Resolution, up to 1280 x 1024 # Manual Focus/Zoom Adjusting 2000 ANSI Lumens Brightness (minimum) Contrast 2000:1 Projector Display System LDP Aspect Ratio 4:3 Full Screen Input Terminals 1x Video (RCA), 1x S-Video D-sub 15-pin, 1x Mini Stereo Jack Wireless PC Link USB Mouse Control Built-In Speaker Keystone Correction (+/-) 15 degree Ceiling Mountable Remote Control Lamp life (minimum) 2000-hour 1 x Spare Bulb Tripod Roller Wall Screen minimum 1.8mx1.8m Ceiling Mount Kit Carrying case Cables to work in connection with workstation POWER CHARACTERISTICS: - Voltage 220-240V Frequency 50-60Hz Internal power supply Power cord appropriate for Serbian electrical sockets |  |  |  |
| 1.6 | Presenter 1, incl 2 ceiling mics and 1 camera (Minimum Specs) State Manufacturer: State Model: State Country of Origin: ITU H.323 and H.320 compliant Bandwidth • Maximum Data Rate IP and Serial/V.35: Up to 2 Mbps • Maximum Data Rate ISDN: Up to 2 Mbps Video Standards & Protocols • H.261, Annex D • H.263+ Annexes: F, I, J, L, N, T • H.263++ Annexes: W • H.264 • ITU 60-fps full screen – Pro-Motion • Intelligently selects frame rate for best performance video • 30 fps at 56 kbps up to 2 Mbps • 60 fields per second up to 2 Mbps Video Inputs: • 1 x S-Video; Professional Y/C BNCs (main video camera) - 1 x DB15; PowerCam and PowerCam Plus power, PTZ control, IR, mic input • 1 x S-Video; 4-pin mini DIN (second camera with PTZ control) • 1 x S-Video; 4-pin mini DIN (VCR or DVD player) • 1 x VGA (Content input from laptop) Serial Data Port: • 2 x DB9 • Control port for custom integration with remote devices such as Crestron® and AMX® control systems • Auxiliary camera control Video Outputs: • 1 x S-Video; Professional Y/C BNCs (main display) • 1 x S-Video; 4-pin mini DIN (second display) • 1 x S-Video; 4-pin mini DIN (VCR or DVD player) • 1 x VGA (Content or video display) Video Formats • NTSC/PAL • Graphics: XGA, SVGA, VGA People Video Resolution • Pro-Motion interlaced video (60/50 fields full-screen video for NTSC/PAL) • 2CIF (352 x 480)- Pro-Motion H.264 • 2SIF (352 x 576)- Pro-Motion H.264 • 4SIF (704 x 480) • 4CIF (704 x 576) • SIF (352 x 240) • CIF (352 x 288) Content Video Resolution • XGA (1024 x 768), SVGA (800 x 600), VGA (640 x 480) for or Content on 2nd VGA Displays (when used with primary NTSC/PAL display) • People video support for 4CIF, 4SIF, 2CIF, 2SIF, CIF and SIF on 2nd VGA display • Single monitor graphics display (1024 x 768) • Up to 4CIF for Content on NTSC/PAL displays Audio Standards & Protocols •14 kHz bandwidth on IP, ISDN, and IP/ISDN mixed calls • 7 kHz bandwidth with G.722, G.722.1 • 3.4 kHz bandwidth with G.711, G.728, G.729A • Full-duplex digital audio • Instant Adaptation Echo Cancellation • Automatic Gain Control (AGC) – Voice activated • Automatic Noise Suppression (ANS) Audio Inputs: • 1 x Conference link - Supports up to (3) microphones • 2 x RCA/Phono, line level input for VCR, DVD player or audio mixer • 2 x Phoenix connectors; balanced line level or direct microphone inputs with 48V Phantom power • 1 x RJ-11 for analog speaker telephone Audio Outputs: • 2 x Phoenix connectors; balanced line level output • 2 x RCA/Phono, Line Level output for VCR record • 2 x RCA/Phono, Line Level output for speakers Other ITU-Supported Standards • H.221 communications • H.224/H.281 far-end camera control • Annex Q standard for FECC in H.323 calls • H.225, H.245, H.241, H.331 • H.239 People+Content Network Interfaces Supported • SIP Support, IP (LAN, DSL, cable modem) • Single 10/100 Ethernet port (10 bps/100Mbps/Auto) • Optional ISDN QBRI (Basic Rate Interface) Module • Optional ISDN PRI (Primary Rate Interface) Module T1/E1 • Integration with Cisco Systems® CallManager Version 4.0 Security • KG-194/KIV-7 encryptor support with on screen and address book dialing • Secure password authentication • Administrator password • Encrypted password for Web access • Advanced Encryption Standard (AES) • AES software encryption on ISDN, IP and Serial/V.35 up to 2 Mbps User Interface • User-friendly graphical interface • Up to (99) user defined camera presets • Dual-Monitor Emulation • Diagnostics and software upgrades via PC, LAN • System configuration from Web Collaboration Solutions • People and Content (PC, VCR/DVD, Doc cam) displayed simultaneously • Show content from laptop/PC over IP network connection • Adjustable bandwidth for content - 10% Content, 90% People - 50% Content, 50% People - 90% Content, 10% People • System must include Internal Multipoint Feature (up to 6 callers, 5 remote and 1 local) System also must include: - one main external camera - People+Content Interface as well as - system must be 19” rack mountable # Embedded Multipoint software ## Extend the reach to more locations - Mixed protocol dialing - IP/ISDN/Serial - Continuous Presence, Voice Activated Switching or Presenter modes - Connect up to six participants - voice or video - Supports content sharing from any endpoint in the call 2 x Microphone array • Electronics enclosure with ceiling mounting hardware (UL 2043 compliant and suitable for use in air-handling spaces) • Microphone interface cable • Conference Link cable • Wall plate Digital Microphone Specifications • 360 degree directional pickup • Three cardioids elements per microphone - Elements spaced 120 degrees apart • Each Ceiling Microphone Array covers a 30' diameter or 15’ radius area • Total coverage area for a single Ceiling Microphone Array must be at least 700 square feet Audio Features: • Dynamic microphone steering • Full-duplex digital audio • Instant Adaptation Echo Cancellation • Automatic Noise Suppression (ANS) • Automatic Gain Control (ACG) • must be compatible with videoconferencing systems 1 x Camera - Silent, motorized pan-tilt-zoom camera - Embedded microphones enable pointing to presets - Built-in IR receiver - Intelligently points the camera to frame whichever meeting participants are speaking, anywhere in the room - Automatically locates and frames multiple speakers in view when more than one person is speaking - Designed for flat screen displays Enables organizations to conduct video meetings without moving the camera manually |  |  |  |
| 1.7 | Presenter 2, incl 2 ceiling mics and 1 camera (Minimum Specs) State Manufacturer: State Model: State Country of Origin: ITU H.323 and H.320 compliant Bandwidth • Maximum Data Rate IP and Serial/V.35: Up to 2 Mbps • Maximum Data Rate ISDN: Up to 2 Mbps Video Standards & Protocols • H.261, Annex D • H.263+ Annexes: F, I, J, L, N, T • H.263++ Annexes: W • H.264 • ITU 60-fps full screen • 30 fps at 56 kbps up to 2 Mbps • 60 fields per second up to 2 Mbps Video Inputs: • Integrated main camera • 1 x S-Video; 4-pin mini DIN (Document camera, VCR or second camera) • 1 x Composite; RCA/Phono (Document camera, VCR or second camera) RS-232 Data Port: • 1 x 8-pin mini DIN • Control port for custom integration with remote devices such as Crestron and AMX control systems • Communication port for transmission of serial data (i.e. medical devices) Video Outputs: • 1 x S-Video; 4-pin mini DIN (Main monitor) • 1 x Composite; RCA/Phono (Main monitor) • 1 x S-Video (Second monitor) • 1 x Composite; RCA/Phono (Second monitor) • 1 x VGA (Second graphics display) Video Formats • NTSC/PAL • Graphics: XGA, SVGA, VGA Integrated Main Camera • pan/tilt/zoom (PTZ) • 65° field of view • Tilt Range: +/- 25° (Up/Down) • Pan Range: +/- 100° (Left/Right) • Total Field of View: 265° • 12x Zoom; f-4.2 to 42mm • F=1.85 to 2.9 mm • Auto focus • Automatic white balance • Camera presets - local and far-end (up to 99) • Standards-based, far-end camera control (FECC) People Video Resolution • 4SIF (704 x 480) • 4CIF (704 x 576) • SIF (352 x 240) • CIF (352 x 288) • Choice of 4:3 or 16:9 display aspect ratios • Display People on VGA Second Monitor Content Video Resolution • XGA (1024 x 768), SVGA (800 x 600), VGA (640 x 480) for graphic displays • People video support for 4CIF and SIF on second VGA display • Up to 4CIF for NTSC/PAL displays Audio Standards & Protocols • 14 kHz bandwidth with G.722.1 Annex C • 7 kHz bandwidth with G.722, G.722.1 • 3.4 kHz bandwidth with G.711, G.728, G.729A Audio Features • Full-duplex digital audio • Instant Adaptation Echo Cancellation • Automatic Gain Control (AGC) – Voice activated • Automatic Noise Suppression (ANS) • Audio Mixer (Mic, VCR, line-in) Audio Inputs: • 1 x Conference Link - Supports up to (3) microphones • 1 x RCA input for stereo or mono inputfrom VCR • 1 x RCA input for stereo or mono input from external mixer Audio Outputs: • 2 x RCA for stereo VCR record (or stereo for auxiliary speakers) Digital Tabletop Microphone • 360° voice pick up • Three cardioid elements per microphone • Integrated mute button Integrated Speaker and Subwoofer Other ITU-Supported Standards • H.221 communications • H.224/H.281 far-end camera control • Annex Q standard for FECC in H.323 calls • H.225, H.245, H.241, H.331 • H.239 People+Content • H.231 in multipoint calls • H.233, H.234, H.235V3 encryption standards Network Interfaces Supported and features • SIP Support, IP (LAN, DSL, cable modem) • Single 10/100 Ethernet port (10 Mbps/100Mbps/Auto) • Optional ISDN QBRI (Basic Rate Interface) Module • Optional ISDN PRI (Primary Rate Interface) Module T1/E1 • Integration with Cisco Systems' CallManager Version 4.0 • Audio & Video Error Concealment over IP, ISDN and mixed calls • Integrated Web Interface. • Internal Multipoint Features (Optional) • Password protection for incoming calls • Cascading for up to 10 conference participants • Secure password authentication • Administrator password • Advanced Encryption Standard (AES) User Interface • User-friendly graphical interface • Dual Monitor Emulation • Recent Calls Log – Records last 99 incoming and outgoing calls • Call Detail Record (CDR) - Reports all incoming and outgoing calls along with call statistics - Show content from laptop/PC over IP network connection (Optional) 2 x Microphone array • Electronics enclosure with ceiling mounting hardware (UL 2043 compliant and suitable for use in air-handling spaces) • Microphone interface cable • Conference Link cable • Wall plate Digital Microphone Specifications • 360 degree directional pickup • Three cardioids elements per microphone - Elements spaced 120 degrees apart • Each Ceiling Microphone Array covers a 30' diameter or 15’ radius area • Total coverage area for a single Ceiling Microphone Array must be at least 700 square feet Audio Features: • Dynamic microphone steering • Full-duplex digital audio • Instant Adaptation Echo Cancellation • Automatic Noise Suppression (ANS) • Automatic Gain Control (ACG) • must be compatible with videoconferencing systems 1 x Camera - Silent, motorized pan-tilt-zoom camera - Embedded microphones enable pointing to presets - Built-in IR receiver - Intelligently points the camera to frame whichever meeting participants are speaking, anywhere in the room - Automatically locates and frames multiple speakers in view when more than one person is speaking - Designed for flat screen displays Enables organizations to conduct video meetings without moving the camera manually |  |  |  |
| 1.8 | Recording and Streaming Server (Minimum Specs) State Manufacturer: State Model: State Country of Origin: Video Standard • H.261 • H.263 • H.263+ • H.264 Video Resolution • QCIF • CIF • SIF • 4CIF • XGA • VGA Frame Rates • Up to 30 Frames per second Audio Standard • G.711a • G.711u • G.722 • G.728 Data Standard • H.239 Network • H.323 v4 Conference Bandwidth • 128K-1,920Kbps Recording • Compatible with H.323 MCU and Endpoints • Single point recording • Point to point recording • Multi-point recording • Synchronized video and content Recording Formats • ASF for Windows Media Player and Real Player Playback • Live Streaming via the web • Archived Playback to H.323 Endpoint MCU or gateway. • Windows Media player and Real Player • Standard ASF format Streaming • Unicast Capacity • internal disk for up to 900 hours of recording • must support up to 2 concurrent video conferencing recording sessions Management • Embedded web server that enables full control, configuration and monitoring of system and conferences • Endpoint and MCU use FECC menu to control recording and viewing. • Direct E-Mail link to an archive Security • Administrator and user passwords • User and Endpoint viewing and recording rights authorization • Terminal identification by IP, E.164 address or H.323 alias • Terminal recording and viewing rights authorization based on the identification Control • Remote control via H.323 terminal • FECC • DTMF Automatic Backup and Delete • System utilities for automatic content management and archiving Network Interfaces • RJ45 Ethernet System Configuration • Processor, DualCore (Muliti Core) technology, 2,66 GHz, 2MB L2 Cache • 2 GB RAM • 250GB Hard Disk Operating System • Windows® XP • 19” rack mountable |  |  |  |
| 1.9 |  |  |  |  |

| Warranty for LOT 1 Please confirm 2 - year warranty provision and provide the following information in your offer |  |  |
| --- | --- | --- |
| Software updates (new releases including product upgrades, maintenance releases and patches) both on-line and off-line. |  |  |
| Support service for product specific questions and instructions on installing and operating the software |  |  |
| Response Time | Solution Time | Repair and/or replacement Time |
| 24 hours | 48 hours | 72 hours |
| Local Agents details |  |  |
| Name: |  |  |
| Address: |  |  |
| Tel/Fax/: |  |  |
| Email: |  |  |

| LOT 2: Computing Equipment with peripherals for Distance Learning |  |  |  |  |
| --- | --- | --- | --- | --- |
| Item Number | Specifications | Specifications Offered | Notes, remarks, ref to documentation | Evaluation Committee’s notes |
| 2.1 | Desktop Computer + Monitor (Minimum Specification) State Manufacturer: State Model: State Country of Origin: Small Desktop Processor, DualCore (Muliti Core) technology, 2,66 GHz, 2MB L2 Cache, 64-bit Instruction Set Support 800MHz FSB 2GB DDR2- 667MHz SDRAM 160GB, 7200rpm, SATA DVD+/-RW DL 1 x PCI expansion slots 1xPCIe expansion slots 1 x PCIex16 expansion slots Sockets: parallel, serial, 6xUSB2.0 Chassis intrusion switch Integrated Video Card is accepted Full duplex sound card, Line in, Mic in, sound and mic on front 10/100/1000 Mbit/s Ethernet Integrated Network Interface Card, Wake-on LAN, Alert-on LAN Keyboard Yu set 2 button optical mouse with wheel and mouse mat, Antivirus personal, 2 Year Licence, pro-active Factory Pre-installed MS Windows Vista Business Edition Microsoft Office 2007 Professional Offered configuration must be Vista certified. (Link from www.microsoft.com must be provided as a proof) Environmental compatibility: Energy Star 4.0 or Blue Angel Rescue and recovery software Screw less design for adapter cards, and drives POWER CHARACTERISTICS: Voltage 220-240V Frequency 50-60Hz Auto sensing Power cord appropriate for the Serbian sockets 17” TFT Monitor Resolution: 1280x1024@75Hz Visible screen diagonal: 17” Pixel Pitch: 0.264 mm Brightness: 300 cd/m2 Contrast: 700:1 Minimum Viewing angle: Horizontal 140°, Vertical 135° Response time: Maximum 5ms Plug & Play: Video input: 1 x D-SUB analog or DVI-D digital TCO'99 or TCO'03 POWER CHARACTERISTICS: Voltage 220-240V Frequency 50-60Hz Power cord appropriate for the Serbian sockets |  |  |  |
| 2.2 | Audio/Video Workstation + Monitor (Minimum Specification) State Manufacturer: State Model: State Country of Origin: Tower case, 2x Processor, DualCore (Multi Core) technology, 2,66 GHz, 4MB L2 Cache, 64-bit data paths and registers 1.33GHz, 64-bit dual independent frontside buses 4GB DDR2- 667 DDR2 SDRAM , 4x 500GB Serial ATA 3Gb/s, 7200-rpm, 8MB cache DVD+/-RW DL Double-wide 16-lane PCI Express graphics slot PCIe 3D SVGA card with 512MB of GDDR3 SDRAM, one single-link DVI port, and one dual-link DVI port 2x PCIe expansion slots 6xUSB, Bluetooth 2.0, FireWire 800, FireWire 400 Full duplex sound card, Line in, Mic in, sound and mic on front 2x 10/100/1000 Mbit/s Ethernet Integrated Network Interface Keyboard, Optical mouse with wheel and mouse mat, Antivirus personal, 2 Year Licence, pro-active Mac OS X v10.4 Tiger (includes Spotlight, Dashboard, Mail, iChat AV, Safari, QuickTime, iCal, and other software) Creative Suite 3 Master Collection Final Cut Studio 2 POWER CHARACTERISTICS: Voltage 220-240V Frequency 50-60Hz Auto sensing Power cord appropriate for the Serbian sockets 2 x 23” TFT Monitor Resolution: 1920x1200 Visible screen diagonal: 23” Pixel Pitch: 0.264 mm Brightness: 270 cd/m2 Contrast: 400:1 Minimum Viewing angle: Horizontal 170°, Vertical 170° Response time: Maximum 16ms Plug & Play: POWER CHARACTERISTICS: Voltage 220-240V Frequency 50-60Hz Power cord appropriate for the Serbian sockets |  |  |  |
| 2.3 | Department Blade Data Server (Minimum Specification) State Manufacturer: State Model: State Country of Origin: Enclosure 7 or 10U in 19 ”server Rack Minimum 5 full size(10 half size) height server moduls (blades) Integrated Server Management Two or more cooling fans Power supplies 2 or more, 2000w or more each Server Two (2) Blade Servers Two Dual or Quade core processor 2.33GHz 4MB L2 Cache 4GB Ram Memory 2x73GB Hot Swap HDD SAS 10k rpm Raid controler Integrated video card Dual Gigabit network controler 2 x USB 2.0 Factory Pre-installed MS Windows Server 2003 with 5 CAL installed Drivers for Windows 2003 Server Direct attach storage Rack mount 12 Hot plug drive bays for SATA HDD RAID Controller Host/Backpanel interface 8 x 1000GB SATA HDD POWER CHARACTERISTICS: Redundant Power Supplies Voltage 220-240V Frequency 50-60Hz Power cord appropriate for the Serbian sockets Direct attach storage must be compatible and ready for connection with Blade server |  |  |  |
| 2.4 | Notebook (Minimum Specification) State Manufacturer: State Model: State Country of Origin: Mobile class Processor, 2.6GHz 2MB L2 Cache 15,4” WSXGATFT Monitor 2GB MB DDRAM Minimum 160GB HDD DVD-RW Video Card with 128MB RAM Audio type/controller: AC97 ,headphones/speakers jack, Mic in jack 10/100/1000 Integrated Network Interface Card (RJ45), Integrated Wireless LAN Integrated or removable Bluetooth Internal 56K V92 Modem Interfaces: VGA, 3x USB 2.0, Modem RJ-11 port, Ethernet LAN: RJ-45 port, IEEE 1394, S-Video Out Docking port Lithium Ion battery (Battery life up to3,0 Hrs) Carry Case Optical USB Mouse USB 2.0 Pen (Flash) Memory Drive 2GB Antivirus personal, 2 Year Licence, including Virus Database and Program Update Pre-installed Windows XP Professional SP2 or Windows Vista Business POWER CHARACTERISTICS: Auto sensing AC adapter (100/240 V) for worldwide usage Power cord appropriate for the Serbian sockets User manual, Notebook utilities and drivers for MS Windows XP, MS Windows Vista |  |  |  |
| 2.5 | Managed L3 Network Switch 24*10/100/1000TX (Minimum Specification) State Manufacturer: State Model: State Country of Origin: Min 32Gbps switch fabric 24 port 10/100/1000TX 2 SFP fiber Combo Slots RIP v1/v2 Static IP Multicast Static routes 802.3x Flow Control MAC Address Table Size 8k 802.1X Network Login Support Broadcast Storm Control 256 active VLANs NETWORK MANAGEMENT: Web-based management SNMP support (MIB, Bridge MIB, Remote network MIB) SECURITY: MAC based port security RADIUS and TACACS+ remote authentication; SSL and SSH management encryption STANDARDS AND COMPLIANCE: 10T Ethernet, 100TX Ethernet, 1000T Ethernet, 1000X Ethernet, Link Aggregation, VLAN Tagging, IGMP, BootP, DHCP, RMON Port Mirroring POWER CHARACTERISTICS: Voltage 220-240V Frequency 50-60Hz Internal power supply Power cord appropriate for the Serbian sockets Rack Mounting Kit |  |  |  |
| 2.6 | 3000 VA UPS Rack Mount (Minimum Specification) State Manufacturer: State Model: State Country of Origin: Intelligent Battery Management, Load Meter Automatic Voltage Regulation (AVR) Overload Indicator User Replaceable batteries Nominal input voltage: 220-230 V Input frequency: 50/60 Hz (auto sensing) Input voltage range for main operations: 180 - 280 Output power capacity: 3000VA Waveform type: Sinewave USB/serial connection with PC/Server feature Includes: CD with software, User Manual POWER CHARACTERISTICS: Voltage 220-240V Frequency 50-60Hz Internal power supply Power cord appropriate for the Serbian sockets |  |  |  |
| 2.7 | 42U Stand-alone Cabinet Rack (Minimum Specification) State Manufacturer: State Model: State Country of Origin: 42U Cabinet Rack, load-bearing cabinet structure made of extruded aluminum or still sections, cast corner parts, removable sides made of steel plate, lockable latches, perforated steel plate top, with minimum 3 ventilators. Transparent or perforated front door, lockable, back ventilated door lockable, ventilated base with adjustable leveling feet, built-in front and back trays for 19” devices, power supply with 2x6 sockets with UPS option, floor panel with patch guide, pre-assembling, packing and accessory pack (var. installation material, manual). Keyboard, optical scroll mouse, 15” TFT monitor, rack-mountable Administration console for min. 2 servers Installation and configuration of all containing equipment |  |  |  |
| 2.8 | Departmental Laser Printer (Minimum Specification) State Manufacturer: State Model: State Country of Origin: 33 ppm A4 capable True 1200 x 1200 dpi Duplex included A4, A5, A6, B5, B6, C5/DL Envelopes 500 Sheet Input PCL 6, PCL 5e, PostScript Level 3 emulation Toner cartridge for printing 12,000 pages (spare + shipped) Standard Ports: USB, 10/100Mbit Fast Ethernet Duty Cycle: 100,000 pages per month All necessary cables Driver Software for MS Windows XP, MS Vista Full Documentation POWER CHARACTERISTICS: Voltage 220-240V Frequency 50-60Hz Internal power supply Power cord appropriate for the Serbian sockets |  |  |  |
| 2.9 | A4 InkJet Printer (Minimum Specification) State Manufacturer: State Model: State Country of Origin: Black print 1200 x 1200 dpi Black:20 ppm A4 capable Color print Color:20ppm A4 capable 1200 x 1200 dpi 4800x1200 dpi or better enxanced/optimized printing resolution A4, A5, Envelopes etc. 120 Sheet Input Standard Ports: USB All necessary cables Driver Software for MS Windows XP, MS Vista Full Documentation POWER CHARACTERISTICS: Voltage 220-240V Frequency 50-60Hz Internal power supply Power cord appropriate for the Serbian sockets |  |  |  |
| 2.10 | A4 Scanner (Minimum Specification) State Manufacturer: State Model: State Country of Origin: - 4800 x 4800 dpi, 48 bit - A4 flat bed scanner USB interface Driver Software for MS Windows XP, MS Windows Vista |  |  |  |

| Warranty for LOT 2 Please confirm 2 – year warranty provision and provide the following information in your offer |  |  |
| --- | --- | --- |
| Software updates (new releases including product upgrades, maintenance releases and patches) both on-line and off-line. |  |  |
| Support service for product specific questions and instructions on installing and operating the software |  |  |
| Response Time | Solution Time | Repair and/or replacement Time |
| 24 hours | 48 hours | 72 hours |
| Local Agents details |  |  |
| Name: |  |  |
| Address: |  |  |
| Tel/Fax/: |  |  |
| Email: |  |  |

- ANNEX II b:

- Distribution / Delivery/ List  for Lot 1 and Lot 2 respectively