# ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Contract title : Supply of Supply of Equipment for the Danube River Information Services (RIS)			p 1 /…

Publication reference : EuropeAid/127790/C/SUP/RS

Column 1-2 should be completed by the Contracting Authority

Column 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

Lot 1: General IT equipment

| 1. Item Number | 2. Specifications | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| S_H_1.1 | Hull Data Base Server |  |  |  |
|  | Features: |  |  |  |
|  | - 19” rack mount case max. 2 height units |  |  |  |
|  | - Redundant power supply units for 230 VAC input voltage |  |  |  |
|  | - 2 x min. Quad core CPU 4x2 GHz |  |  |  |
|  | - Min. 4 GB DDR2 RAM (expandable up to 16 GB) |  |  |  |
|  | - Min. 2x300 GB SAS HDD (Raid level 1) |  |  |  |
|  | - Internal DVD±R DL, CD±RW |  |  |  |
|  | - Min. 2 x 10/100/1000 MBit Ethernet adapter |  |  |  |
|  | - Min. 4 x USB v2.0 ports (min. 1 Front USB) |  |  |  |
| S_H_1.2 | Minimum Hull DB Server |  |  |  |
|  | Features: |  |  |  |
|  | - 19” rack mount case max. 2 height units |  |  |  |
|  | - Redundant power supply units for 230 VAC input voltage |  |  |  |
|  | - 2 x min. Dual core CPU 2x2,5 GHz |  |  |  |
|  | - Min. 4 GB DDR2 RAM (expandable up to 16 GB) |  |  |  |
|  | - Min. 2x300 GB SAS HDD (Raid level 1) |  |  |  |
|  | - Internal DVD±R DL, CD±RW |  |  |  |
|  | - Min. 2 x 10/100/1000 MBit Ethernet adapter |  |  |  |
|  | - Min. 4 x USB v2.0 ports (min. 1 Front USB) |  |  |  |
| S_H_1.3 | UPS |  |  |  |
|  | Features |  |  |  |
|  | - Output power capacity: min. 6000VA |  |  |  |
|  | - Optional Battery pack (if necessary) |  |  |  |
|  | - Nominal output voltage: 230 VAC |  |  |  |
|  | - Nominal input voltage: 230 VAC |  |  |  |
|  | - Output frequency: 50 Hz |  |  |  |
|  | - Input frequency: 50 Hz |  |  |  |
|  | - Standard output for backup power: min. 4 IEC 320 C13 outlets |  |  |  |
|  | - Two separate controllable power segments |  |  |  |
|  | - Automatic internal bypass |  |  |  |
|  | - Maintenance free batteries |  |  |  |
|  | - Automatic self test |  |  |  |
|  | - Cold start capability |  |  |  |
|  | - Ethernet SNMP interface |  |  |  |
|  | - Drivers for Windows and Linux OS |  |  |  |
|  | - Software for automatic shut-down and auto restart of the protected computer systems |  |  |  |
|  | - Software that enables remote monitoring of the UPS |  |  |  |
|  | - Power cord |  |  |  |
|  | - 2 IEC 320 C13 power cords for external components |  |  |  |
|  | - 2 power extension units, 19” rack mount with min. 4 sockets each |  |  |  |
|  | - Cat.5e patch cable, flexible, sealed connectors, length 5m |  |  |  |
|  | - One year on site warranty |  |  |  |
| S_H_1.4 | KVM TFT unit |  |  |  |
|  | Features: |  |  |  |
|  | - 19” rack mount |  |  |  |
|  | - Max. 2 height units folded |  |  |  |
|  | - Min. 15” TFT display |  |  |  |
|  | - Keyboard plus trackball |  |  |  |
|  | - Min. 4 KVM ports |  |  |  |
| S_H_1.5 | Network Switch |  |  |  |
|  | Features |  |  |  |
|  | - 19” rack mount |  |  |  |
|  | - 10/100/1000MBit |  |  |  |
|  | - Min. 8 ports |  |  |  |
| S_H_1.6 | Firewall |  |  |  |
|  | Features |  |  |  |
|  | - 19” rack mount |  |  |  |
|  | - Min. 150 Mbps firewall |  |  |  |
|  | - IP sec VPN |  |  |  |
|  | - Min. 8 user ports |  |  |  |
| S_H_1.7 | 19” rack |  |  |  |
|  | Features |  |  |  |
|  | - 19” inch form factor |  |  |  |
|  | - Enclosure frame made of stainless steel |  |  |  |
|  | - Aluminium side panels left and right, powder coated |  |  |  |
|  | - Ventilated aluminium front and back door, powder coated |  |  |  |
|  | - Aluminium roof with cable outlet, powder coated |  |  |  |
|  | - Min. 42 height units |  |  |  |
|  | - Min. depth 1000mm |  |  |  |
|  | - Min. payload 1000kg |  |  |  |
|  | - Earthing kit |  |  |  |
|  | - All necessary rails, screws and other mounting equipment for installation of the Hull DB Servers components |  |  |  |
| S_H_1.8 | Hull DB work stations |  |  |  |
|  | Features: |  |  |  |
|  | - Mini/Midi tower Case |  |  |  |
|  | - Min. Dual Core CPU, 2x2,5 GHz |  |  |  |
|  | - 3 GB DDR2 RAM (expandable up to 4 GB) |  |  |  |
|  | - Min. 2x500 GB HDD (SATA Raid Level 1) |  |  |  |
|  | - internal DVD±R DL, CD±RW |  |  |  |
|  | - 10/100/1000 MBit Ethernet adapter |  |  |  |
|  | - 4 x USB v2.0 ports (min. 2 Front USB) |  |  |  |
|  | - Graphics adapter min. 256MB dedicated RAM, DVI interface |  |  |  |
|  | - Keyboard (USB or PS/2) |  |  |  |
|  | - Optical mouse with scroll wheel (USB or PS/2) |  |  |  |
|  | Accessories: |  |  |  |
|  | - Cat.5e patch cable, flexible, sealed connectors, length 5m |  |  |  |
|  | - Power cord |  |  |  |
|  | Operating System: |  |  |  |
|  | - Microsoft Windows XP Professional (preferred) or Windows Vista Business - preinstalled and licensed, - CD and documentation included - including the latest service pack, patches and all critical updates - English language user interface - Fully licensed recovery CD - All device drivers on CD |  |  |  |
|  | Microsoft Office Basic 2007 |  |  |  |
|  | - Preinstalled - English language user interface |  |  |  |
|  | Adobe Acrobat Reader (latest version) |  |  |  |
|  | - Preinstalled |  |  |  |
| S_H_1.9 | 22” TFT LCD screens |  |  |  |
|  | Features: |  |  |  |
|  | - min. 22” screen size |  |  |  |
|  | - min. resolution 1400x1050 pixel |  |  |  |
|  | - reaction time <10ms |  |  |  |
|  | - contrast > 700:1 |  |  |  |
|  | - brightness > 250cd/m² |  |  |  |
|  | - viewing angle >= 160/160 |  |  |  |
|  | - integrated loudspeakers |  |  |  |
|  | - DVI video input |  |  |  |
|  | Accessories: |  |  |  |
|  | - DVI video cable, shielded, min. 2 m length |  |  |  |
|  | - Power cord |  |  |  |
|  | - Audio cable, min 2m length |  |  |  |
| S_H_1.10 | Laptop |  |  |  |
|  | Features: |  |  |  |
|  | - Min. 15” screen, non-glare |  |  |  |
|  | - Min. resolution 1280x800 pixel |  |  |  |
|  | - Min Dual Core CPU, 2x2 GHz |  |  |  |
|  | - 2 GB DDR2 RAM (expandable up to 4 GB) |  |  |  |
|  | - Min 160 GB HDD |  |  |  |
|  | - internal DVD±R DL, CD±RW |  |  |  |
|  | - 10/100/1000 MBit Ethernet adapter |  |  |  |
|  | - Integrated wireless LAN 802.11 b/g/n |  |  |  |
|  | - Min 4 x USB 2.0 ports |  |  |  |
|  | - PCMCIA or PC Express card slot |  |  |  |
|  | - VGA or DVI out |  |  |  |
|  | - Integrated 3 in 1 card reader |  |  |  |
|  | - Li-Ion battery for min. 2 hours of independent operation |  |  |  |
|  | - Integrated track pad |  |  |  |
|  | Accessories: |  |  |  |
|  | - Cat.5e patch cable, flexible, sealed connectors, length 5m |  |  |  |
|  | - AC adapter with power cord |  |  |  |
|  | - Optical mouse with scroll wheel (USB) |  |  |  |
|  | - Carrying case |  |  |  |
|  | Operating System: |  |  |  |
|  | - Microsoft Windows XP Professional (preferred) or Windows Vista Business - preinstalled and licensed, - CD and documentation included - including the latest service pack, patches and all critical updates - English language user interface - Fully licensed recovery CD - All device drivers on CD |  |  |  |
|  | Microsoft Office Basic 2007 |  |  |  |
|  | - Preinstalled - English language user interface |  |  |  |
|  | Adobe Acrobat Reader (latest version) |  |  |  |
|  | - Preinstalled |  |  |  |

ENC Production and Distribution System

| 1. Item Number | 2. Specifications | 3. Specifications Offered |  | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- | --- |
| S_ENC_1.1 | Mobile work station |  |  |  |  |
|  | Features: |  |  |  |  |
|  | - Processor: Intel Core 2 Duo T9300 (2.50 GHz, 6 MB L2, 800 MHz FSB) - RAM 2GB DDR2 667MHz, - HDD 200GB 7200rpm SATA, - DVD+/-RW Drive - Display 17" TFT WUXGA - Graphics: NVIDIA Quadro FX 1600M 512 MB - Network adapter, 56K fax modem, Intel Wireless WiFi Link 4965AGN, Bluetooth - High Definition Audio w/24-bit DAC (integrated stereo speakers and microphone) - Lithium-Ion 8 cells battery (73 Wh) - Weight up to 3,5 kg - Windows XP Professional (preferred) or Windows Vista Business - English language user interface |  |  |  |  |
| S_ENC_1.2 | Workstation |  |  |  |  |
|  | Features: |  |  |  |  |
|  | - 2.40 GHz Core 2 Quad Q6600 - 4GB DDR2-800 ECC - HDD 500GB - DWD-RW NVIDIA Quadro FX1700 512MB, power supply unit at least 475W, or equivalent - Windows XP Professional (preferred) or Windows Vista Business - English language user interface |  |  |  |  |
| S_ENC_1.3 | Monitor |  |  |  |  |
|  | Features: |  |  |  |  |
|  | - 28" wide 3ms response time, DVI 1920x1200, 800:1,500cd, Audio, VGA, HDMI, Composite, S-Video, Component, or equivalent |  |  |  |  |
| S_ENC_3.1 | Operator desktop PC for multi-beam Echosounder Minimum technical specification for typical desktop PC: - Intel Dual-Core 3 GHz) - RAM 1 GByte - 80Gb hard disk - additional portable USB hard disk 160Gb - DVD+/-R - Graphics board (dual-head) - 100 MBit LAN - Mouse or Trackball Desktop casing (anti-vibration kit for marine applications) |  |  |  |  |
| S_ENC_3.2 | Operator notebook PC for multi-beam Echosounder Minimum technical specification: - Pentium M processor or equivalent 2 GHz or more - RAM 1 GByte - 80 GB hard disk - additional portable USB hard disk 160GB - DVD+/-R - 15.0” XGA display with 1024 x 768 resolution and 16.7 million colours - 100 MBit LAN - Mouse or Trackball |  |  |  |  |
| S_ENC_3.3 | Auxiliary PC components for multi-beam Echosounder - 19” TFT monitor (two units), resolution 1280 x 1024 - USB hard disk in external casing 250 GB - Helmsman kit for 20 m cable distance: - VGA splitter - High quality VGA cable (max. 20 m) - Helmsman kit for 150 m cable distance - VGA splitter - VGA Extender Set - via CAT5 cable - max. 150 m cable length - max. display resolution 1280 x 1024 - CAT5 cable optional upon request |  |  |  |  |

D4D International Data Exchange System

| 1. Item Number | 2. Specifications | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| S_D4D_1.2 | Workstation PC |  |  |  |
|  | Features: - CPU: 2.40GHz Core 2 Quad Q6600 - Memory: 4GB DDR2-800 ECC - Storage: HDD 500GB - DVD-RW NVIDIA Quadro FX1700 512MB, power supply unit at least 475W, or equivalent |  |  |  |
|  | Interfaces: - Keyboard - PS/2 optical mouse with scroll wheel, mouse pad - 10/100/1000Mbit Network adapter - Minimum 4 USB v2.0 ports |  |  |  |
|  | Operating System: Microsoft Windows XP Professional (preferred) or Windows Vista business - Preinstalled and licensed CD and documentation included - including the latest service pack, patches and all critical updates - All device drivers on CD - English language user interface |  |  |  |
|  | Software: Microsoft Office Basic 2007 - English language user interface - preinstalled |  |  |  |
|  | Warranty: - One (1) year on site warranty |  |  |  |
| S_D4D_1.3 | Monitor TFT 24 inch |  |  |  |
|  | Features: - 24 inch wide 5ms response time, DVI 1920x1200, 1000:1, 400cd/m², Audio, VGA, HDMI, USB port |  |  |  |
| S_D4D_1.4 | Mobile work station |  |  |  |
|  | Features: |  |  |  |
|  | - Processor: Intel Core 2 Duo T9300 (2.50 GHz, 6 MB L2, 800 MHz FSB) - RAM 2GB DDR2 667MHz, - HDD 200GB 7200rpm SATA, - DVD+/-RW Drive - Display 17" TFT WUXGA - Graphics: NVIDIA Quadro FX 1600M 512 MB - Network adapter, 56K fax modem, Intel Wireless WiFi Link 4965AGN, Bluetooth - High Definition Audio w/24-bit DAC (integrated stereo speakers and microphone) - Lithium-Ion 8 cells battery (73 Wh) - Weight up to 3,5 kg - Windows XP Professional (preferred) or Windows Vista Business - English language user interface |  |  |  |

| 1. Item Number | 2. Specifications | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| S_LMS_1.1 | Local server |  |  |  |
|  | Chassis: |  |  |  |
|  | Rack mounted with rack height 2U |  |  |  |
|  | CPU |  |  |  |
|  | Intel Dual Core or 2 Core Duo Processor with min. 2,0 GHz |  |  |  |
|  | Memory: |  |  |  |
|  | Minimum 2 GB DDR2 RAM expandable up to min. 4 GB |  |  |  |
|  | Hard drive: |  |  |  |
|  | - Minimum 2 x 250 GB SAS, Hot-Swap - 8-Port SAS Controller, Support for Raid -0.-1.-5 |  |  |  |
|  | Cooling: |  |  |  |
|  | Minimum 2 cooling fans, Hot-Swap |  |  |  |
|  | Power supply: |  |  |  |
|  | Redundant power supply, Hot-Swap |  |  |  |
|  | Interfaces: |  |  |  |
|  | - Min. 2 x 10/100/1000Mbit Network adapter - Min. 2 USB v2.0 ports |  |  |  |
|  | Peripheral equipment: |  |  |  |
|  | - Keyboard - PS/2 optical mouse with scroll wheel |  |  |  |
|  | CD/DVD Drives: |  |  |  |
|  | DVD +/- RW Drive (read/write CD and DVD) |  |  |  |

RIS Centre

| 1. Item Number | 2. Specifications | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| S_C_1.1 | Blade Server Equipment |  |  |  |
|  | Blade enclosure: (1 unit) |  |  |  |
|  | - rack mount |  |  |  |
|  | - Min. 8 full height / 16 half height device bays |  |  |  |
|  | - Min. 3 (redundant=6) power supply bays , suitable for 3-phase hot-plug power supply units |  |  |  |
|  | - Max. amount of hot-plug cooling fans (6 minimum) |  |  |  |
|  | - Min. 3 (redundant=6) interconnection bays |  |  |  |
|  | - 1 administrator access bay (redundant =2) |  |  |  |
|  | - Min. 2 SAS connectors |  |  |  |
|  | - Min. 2 10GBit connectors |  |  |  |
|  | - Status display |  |  |  |
|  | Server blades: (4 units) |  |  |  |
|  | - Min. 4 Intel XEON (equivalent or better) Quad Core 2,4 GHz 2x4MB 1066 MHz FSB |  |  |  |
|  | - Min. 16 GB DDR2 RAM, fully buffered |  |  |  |
|  | - Raid level 1 with min. 2 x 146GB SAS HDD |  |  |  |
|  | - Min. 2 GBit Ethernet network adapters |  |  |  |
|  | - Min 2 10GBit server interconnection port |  |  |  |
|  | Network Attached Storage (or blade storage) |  |  |  |
|  | - Min. 2 TB total storage capacity |  |  |  |
|  | - SAS HHD drives |  |  |  |
|  | - Raid level 5 |  |  |  |
|  | - Min. 2 GBit Ethernet network adapters |  |  |  |
|  | - Min. 1 10GBit Ethernet adapter |  |  |  |
|  | - Min. 4 GB internal RAM |  |  |  |
|  | - Min. 1 Dual Core Processor 4x2GHz |  |  |  |
|  | Backup Tape Library |  |  |  |
|  | - 19” rack mount |  |  |  |
|  | - LTO-4 Format |  |  |  |
|  | - 16 cartridges capacity |  |  |  |
|  | - Barcode reader |  |  |  |
|  | - SAS interface |  |  |  |
|  | - Including 32 LTO-4 cartridges 800/1600 |  |  |  |
|  | UPS |  |  |  |
|  | - 19” mounted in suitable rack |  |  |  |
|  | - Output power capacity: min. 30kVA |  |  |  |
|  | - Optional Battery pack (if necessary) |  |  |  |
|  | - Nominal output voltage: 230 VAC |  |  |  |
|  | - Nominal input voltage: 400VAC (3 phase) |  |  |  |
|  | - Output frequency: 50 Hz |  |  |  |
|  | - Input frequency: 50 Hz |  |  |  |
|  | - Standard output for backup power: min. 6 IEC 320 C13 outlets |  |  |  |
|  | - Standard output for backup power: min. 2 IEC 320 C19 outlets |  |  |  |
|  | - Two separate controllable power segments |  |  |  |
|  | - Automatic internal bypass |  |  |  |
|  | - Maintenance free batteries |  |  |  |
|  | - Automatic self test |  |  |  |
|  | - Cold start capability |  |  |  |
|  | - Ethernet SNMP interface |  |  |  |
|  | - Drivers for Windows and Linux OS |  |  |  |
|  | - Software for automatic shut-down and auto restart of the protected computer systems |  |  |  |
|  | - Software that enables remote monitoring of the UPS |  |  |  |
|  | - Power cord |  |  |  |
|  | - 4 IEC 320 C13 power cords for external components |  |  |  |
|  | - 2 power extension units, 19” rack mount with min. 4 sockets each |  |  |  |
|  | - Cat.5e patch cable, flexible, sealed connectors, length 5m |  |  |  |
|  | - One year on site warranty |  |  |  |
|  | Network switch |  |  |  |
|  | - 19” rack mount |  |  |  |
|  | - Min. 24 10/100/1000MBit ports |  |  |  |
|  | - Min. 10 10GBit ports (alternatively additional 10 GBit switch) |  |  |  |
|  | KVM TFT unit |  |  |  |
|  | - 19” rack mount |  |  |  |
|  | - Max. 2 height units folded |  |  |  |
|  | - Min. 17” TFT display |  |  |  |
|  | - Keyboard plus trackball |  |  |  |
|  | - Min. 8 KVM ports |  |  |  |
|  | 19” Server Rack |  |  |  |
|  | - 19” inch form factor |  |  |  |
|  | - Enclosure frame made of stainless steel |  |  |  |
|  | - Aluminium side panels left and right, powder coated |  |  |  |
|  | - Ventilated aluminium front and back door, powder coated |  |  |  |
|  | - Aluminium roof with cable outlet, powder coated |  |  |  |
|  | - Min. 42 height units |  |  |  |
|  | - Min. depth 1000mm |  |  |  |
|  | - Min. payload 1000kg |  |  |  |
|  | - Earthing kit |  |  |  |
|  | - All necessary rails, screws and other mounting equipment for installation of the RIS Centre components |  |  |  |
| S_C_1.2 | High performance work stations – LUWS Type 3 |  |  |  |
|  | Features: |  |  |  |
|  | - Mini/Midi tower Case |  |  |  |
|  | - Quad Core CPU, 4x2,5 GHz |  |  |  |
|  | - 3 GB DDR2-800 RAM (expandable up to 4 GB) |  |  |  |
|  | - 2x500 GB HDD (SATA Raid Level 1) |  |  |  |
|  | 4 of 6 units with - internal DVD±R DL writer 2 of 6 units with - internal Blue-Ray writer |  |  |  |
|  | - 10/100/1000 MBit Ethernet adapter |  |  |  |
|  | - 6 x USB v2.0 ports (min. 2 Front USB) |  |  |  |
|  | - Dual Head graphics adapter min. 512MB dedicated RAM, DVI interface |  |  |  |
|  | - Keyboard (USB or PS/2) |  |  |  |
|  | - Wireless optical mouse with scroll wheel (USB or PS/2) |  |  |  |
|  | Accessories: |  |  |  |
|  | - Cat.5e patch cable, flexible, sealed connectors, length 5m |  |  |  |
|  | - Power cord |  |  |  |
|  | Operating System: |  |  |  |
|  | - Microsoft Windows XP Professional (preferred) or Windows Vista Business - preinstalled and licensed, - CD and documentation included - including the latest service pack, patches and all critical updates - English language user interface - Fully licensed recovery CD - All device drivers on CD |  |  |  |
|  | Microsoft Office Basic 2007 |  |  |  |
|  | - Preinstalled - English language user interface |  |  |  |
|  | Adobe Acrobat Reader (latest version) |  |  |  |
|  | - Preinstalled |  |  |  |
| S_C_1.3 | 24” TFT LCD screens |  |  |  |
|  | Features: |  |  |  |
|  | - min. 24” screen size |  |  |  |
|  | - min. resolution 1400x1050 pixel |  |  |  |
|  | - reaction time <10ms |  |  |  |
|  | - contrast > 700:1 |  |  |  |
|  | - brightness > 250cd/m² |  |  |  |
|  | - viewing angle >= 160/160 |  |  |  |
|  | - integrated loudspeakers |  |  |  |
|  | - DVI video input |  |  |  |
|  | Accessories: |  |  |  |
|  | - DVI video cable, shielded, min. 2 m length |  |  |  |
|  | - Power cord |  |  |  |
|  | - Audio cable, min 2m length |  |  |  |
| S_C_1.4 | A3 network colour laser copier/scanner/printer |  |  |  |
|  | Features: |  |  |  |
|  | - Digital colour laser printer |  |  |  |
|  | - Paper format up to A3 |  |  |  |
|  | - Min. 25 A4 pages per minute B&W |  |  |  |
|  | - Min. 20 A4 pages per minute colour |  |  |  |
|  | - Automatic duplex |  |  |  |
|  | - TCP/IP interface |  |  |  |
|  | - Min 512 MB RAM |  |  |  |
|  | - Min. 2 Trays with min. 250 sheets capacity each |  |  |  |
|  | - 1 manual bypass |  |  |  |
|  | - Automatic sorter |  |  |  |
|  | - Integrated stapler |  |  |  |
|  | - Scan Resolution at least 600x600 dpi |  |  |  |
|  | Interface: |  |  |  |
|  | - USB PC connection port - Microsoft Windows XP/Vista drivers |  |  |  |
|  | Accessories: |  |  |  |
|  | - Power cord - Ethernet cable - Two toner cartridges per colour |  |  |  |
|  | Warranty: |  |  |  |
|  | - One (1) year on-site warranty |  |  |  |
| S_C_1.5 | Uninterrupted power supply (UPS) |  |  |  |
|  | Output power capacity: |  |  |  |
|  | - Output power capacity: min. 2000VA/1200W |  |  |  |
|  | - Nominal output voltage: 230 VAC |  |  |  |
|  | - Nominal input voltage: 230 VAC |  |  |  |
|  | - Output frequency: 50 Hz |  |  |  |
|  | - Input frequency: 50 Hz |  |  |  |
|  | - Standard output for backup power: min. 4 IEC outlets |  |  |  |
|  | - Two separate controllable power segments |  |  |  |
|  | Interface: |  |  |  |
|  | - Ethernet SNMP interface - Microsoft Windows XP/Vista drivers |  |  |  |
|  | Software: |  |  |  |
|  | - Software for automatic shut-down and auto restart of the protected computer systems. - Software that enables remote monitoring of the UPS - Local and remote shutdown - Power management software for UPS for Windows XP/Vista, CD and documentation included - Microsoft Windows XP/Vista drivers |  |  |  |
|  | Interface: |  |  |  |
|  | - Power cord - Cat.5e patch cable, flexible, sealed connectors, length 5m |  |  |  |
|  | Acoustic noise |  |  |  |
|  | - Max. 40 dB |  |  |  |
|  | Warranty |  |  |  |
|  | - One (1) year on-site warranty |  |  |  |
| S_C_1.6 | Presentation Displays |  |  |  |
|  | Features: |  |  |  |
|  | - LCD min. 42” screen size |  |  |  |
|  | - min. resolution 1366x768 pixel |  |  |  |
|  | - reaction time <10ms |  |  |  |
|  | - contrast > 700:1 |  |  |  |
|  | - brightness > 400cd/m² |  |  |  |
|  | - viewing angle >= 170/170 |  |  |  |
|  | - DVI video input |  |  |  |
|  | Accessories: |  |  |  |
|  | - DVI video cable, shielded, min. 4 m length |  |  |  |
|  | - Power cord |  |  |  |
| S_C_1.7 | Data projector |  |  |  |
|  | Features: |  |  |  |
|  | - Min. 4000 ANSI lumen |  |  |  |
|  | - min. native resolution 1024x768 pixel |  |  |  |
|  | - 4:3 native ratio (16:9 compatible) |  |  |  |
|  | - projector min. distance max. 1,2m |  |  |  |
|  | - contrast > 1000:1 |  |  |  |
|  | - integrated loudspeaker |  |  |  |
|  | - Acoustic noise max. 35dB(A) in standard operation |  |  |  |
|  | - Inputs: 2xVGA, 1xHDMI, 1xS-Video, 1xComposite Video, 1 control port, audio in |  |  |  |
|  | - Outputs: 1xVGA, audio out |  |  |  |
|  | - Suitable for ceiling mounting |  |  |  |
|  | Accessories: |  |  |  |
|  | - 2xVGA cable, 5m length plus DVI adapter |  |  |  |
|  | - Cable for remote control |  |  |  |
|  | - Remote control |  |  |  |
|  | - Power cord |  |  |  |
|  | - Spare lamp |  |  |  |
| S_C_1.8 | Mobile Data projector |  |  |  |
|  | Features: |  |  |  |
|  | - Min. 2000 ANSI lumen |  |  |  |
|  | - min. native resolution 1024x768 pixel |  |  |  |
|  | - 4:3 native ratio (16:9 compatible) |  |  |  |
|  | - Max. weight 2 kg |  |  |  |
|  | - contrast > 700:1 |  |  |  |
|  | - integrated loudspeaker |  |  |  |
|  | - Acoustic noise max. 35dB(A) in standard operation |  |  |  |
|  | - Inputs: 1xVGA, 1xS-Video, 1xComposite Video, audio in |  |  |  |
|  | - Outputs: 1xVGA, audio out |  |  |  |
|  | Accessories: |  |  |  |
|  | - 1xVGA cable, 3m length plus DVI adapter |  |  |  |
|  | - Carrying case |  |  |  |
|  | - Remote control |  |  |  |
|  | - Power cord |  |  |  |
|  | - Spare lamp |  |  |  |
| S_C_1.9 | Test Laptop |  |  |  |
|  | Features: |  |  |  |
|  | - Min. 14,1” screen, non-glare |  |  |  |
|  | - Min. resolution 1280x800 pixel |  |  |  |
|  | - Min Dual Core CPU, 2x2 GHz |  |  |  |
|  | - 2 GB DDR2 RAM (expandable up to 4 GB) |  |  |  |
|  | - Min 160 GB HDD |  |  |  |
|  | - internal DVD±R DL, CD±RW |  |  |  |
|  | - 10/100/1000 MBit Ethernet adapter |  |  |  |
|  | - Integrated wireless LAN 802.11 b/g/n |  |  |  |
|  | - Min 4 x USB 2.0 ports |  |  |  |
|  | - PCMCIA or PC Express card slot |  |  |  |
|  | - VGA or DVI out |  |  |  |
|  | - Integrated 3 in 1 card reader |  |  |  |
|  | - Li-Ion battery for min. 2 hours of independent operation |  |  |  |
|  | - Integrated track pad |  |  |  |
|  | Accessories: |  |  |  |
|  | - Cat.5e patch cable, flexible, sealed connectors, length 5m |  |  |  |
|  | - AC adapter with power cord |  |  |  |
|  | - Optical mouse with scroll wheel (USB) |  |  |  |
|  | - Carrying case |  |  |  |
|  | Operating System: |  |  |  |
|  | - Microsoft Windows XP Professional (preferred) or Windows Vista Business - preinstalled and licensed, - CD and documentation included - including the latest service pack, patches and all critical updates - English language user interface - Fully licensed recovery CD - All device drivers on CD |  |  |  |
|  | Microsoft Office Basic 2007 |  |  |  |
|  | - Preinstalled - English language user interface |  |  |  |
|  | Adobe Acrobat Reader (latest version) |  |  |  |
|  | - Preinstalled |  |  |  |
| S_C_1.10 | Ultra portable Laptop |  |  |  |
|  | Features: |  |  |  |
|  | - Max. 12,1” screen, non-glare |  |  |  |
|  | - Min. resolution 1280x800 pixel |  |  |  |
|  | - Min. Dual Core CPU, 2x1 GHz |  |  |  |
|  | - 2 GB DDR2 RAM (expandable up to 4 GB) |  |  |  |
|  | - Min. 60 GB HDD or SSD |  |  |  |
|  | - internal or external DVD±R DL, CD±RW |  |  |  |
|  | - 10/100/1000 MBit Ethernet adapter |  |  |  |
|  | - Integrated wireless LAN 802.11 b/g/n |  |  |  |
|  | - Integrated UMTS/HSDPA modem |  |  |  |
|  | - Min. 3 x USB 2.0 ports |  |  |  |
|  | - PCMCIA or PC Express card slot |  |  |  |
|  | - VGA or DVI out |  |  |  |
|  | - Li-Ion battery for min. 2 hours of independent operation |  |  |  |
|  | - Integrated track pad |  |  |  |
|  | Accessories: |  |  |  |
|  | - AC adapter with power cord |  |  |  |
|  | - Optical mouse with scroll wheel (USB) |  |  |  |
|  | - Carrying case |  |  |  |
|  | Operating System: |  |  |  |
|  | - Microsoft Windows XP Professional (preferred) or Windows Vista Business - preinstalled and licensed, - CD and documentation included - including the latest service pack, patches and all critical updates - English language user interface - Fully licensed recovery CD - All device drivers on CD |  |  |  |
|  | Microsoft Office Basic 2007 |  |  |  |
|  | - Preinstalled - English language user interface |  |  |  |
|  | Adobe Acrobat Reader (latest version) |  |  |  |
|  | - Preinstalled |  |  |  |
| S_C_1.11 | External HDD |  |  |  |
|  | Features: |  |  |  |
|  | - Min. 1 TB storage capacity |  |  |  |
|  | - Fan-less case |  |  |  |
|  | - USB 2.0 connector providing min. 480 MBit/s |  |  |  |
|  | - Ethernet RJ45 connector supporting SMB/DHCP/UPNP, 10/100 MBit/s |  |  |  |
|  | Accessories: |  |  |  |
|  | - AC adapter with power cord |  |  |  |
|  | - USB cable min 3m length |  |  |  |
|  | - Cat.5e patch cable, flexible, sealed connectors, length 5m |  |  |  |
|  | - Necessary drivers for Linux, Windows XP and Windows Vista |  |  |  |
| S_C_1.12 | External DVD/CD Writer |  |  |  |
|  | Features: |  |  |  |
|  | - DVD±R min 18x |  |  |  |
|  | - DVD±R DL min. 8x |  |  |  |
|  | - DVD±RW min. 8x |  |  |  |
|  | - CD-R min. 40x |  |  |  |
|  | - USB 2.0 connector |  |  |  |
|  | - Fire wire (IEEE 1394) connector |  |  |  |
|  | Accessories: |  |  |  |
|  | - USB cable min 3m length |  |  |  |
|  | - Necessary drivers for Linux, Windows XP and Windows Vista |  |  |  |
|  | - CD/DVD burning software |  |  |  |
| S_C_1.13 | External Blue-Ray Writer |  |  |  |
|  | Features: |  |  |  |
|  | - 4x/2x/4x BD-R/BD-RE/BD-ROM |  |  |  |
|  | - HD DVD-ROM optional |  |  |  |
|  | - DVD±RW min. 8x |  |  |  |
|  | - CD-R min. 40x |  |  |  |
|  | - USB 2.0 connector |  |  |  |
|  | - Fire wire (IEEE 1394) connector optional |  |  |  |
|  | Accessories: |  |  |  |
|  | - USB cable min 3m length |  |  |  |
|  | - Necessary drivers for Linux, Windows XP and Windows Vista |  |  |  |
|  | - Suitable Burning software |  |  |  |
| S_C_1.14 | RS 422 to RS 232 converter |  |  |  |
|  | Features: |  |  |  |
|  | - RS 422 input (4 wires) |  |  |  |
|  | - RS 232 output (D-Sub 9 female) |  |  |  |
|  | - 3000 V isolation |  |  |  |
|  | - Supported speed: 1,2, 2,4, 4,8, 9.6, 19,2, 38,4, 57,6 and 115,2 Kbps |  |  |  |
|  | Accessories: |  |  |  |
|  | - RS 232 cable with D-Sub 9 connectors |  |  |  |
| S_C_1.15 | RS 422 to USB converter |  |  |  |
|  | Features: |  |  |  |
|  | - RS 422 input (4 wires) |  |  |  |
|  | - USB output |  |  |  |
|  | - 3000 V isolation |  |  |  |
|  | - Supported speed: 1,2, 2,4, 4,8, 9.6, 19,2, 38,4, 57,6 and 115,2 Kbps |  |  |  |
|  | Accessories: |  |  |  |
|  | - USB cable with USB B output |  |  |  |
| S_C_1.16 | USB to RS 232 converter |  |  |  |
|  | Features: |  |  |  |
|  | - RS 232 input (D-Sub 9) |  |  |  |
|  | - USB B output |  |  |  |
|  | - Supported speed: 1,2, 2,4, 4,8, 9.6, 19,2, 38,4, 57,6 and 115,2 Kbps |  |  |  |
| S_C_1.17 | Training Laptop |  |  |  |
|  | Features: |  |  |  |
|  | - Min. 14,1” screen, non-glare |  |  |  |
|  | - Min. resolution 1280x800 pixel |  |  |  |
|  | - Min. Quad Core CPU, 4x2 GHz |  |  |  |
|  | - Min. 3 GB DDR2 RAM (expandable up to 4 GB) |  |  |  |
|  | - Min. 300 GB HDD |  |  |  |
|  | - internal DVD±R DL, CD±RW |  |  |  |
|  | - 10/100/1000 MBit Ethernet adapter |  |  |  |
|  | - Integrated wireless LAN 802.11 b/g/n |  |  |  |
|  | - Min 4 x USB 2.0 ports |  |  |  |
|  | - PCMCIA or PC Express card slot |  |  |  |
|  | - VGA or DVI out |  |  |  |
|  | - Integrated 3 in 1 card reader |  |  |  |
|  | - Li-Ion battery for min. 2 hours of independent operation |  |  |  |
|  | - Integrated track pad |  |  |  |
|  | Accessories: |  |  |  |
|  | - Cat.5e patch cable, flexible, sealed connectors, length 5m |  |  |  |
|  | - AC adapter with power cord |  |  |  |
|  | - Optical mouse with scroll wheel (USB) |  |  |  |
|  | - Carrying case |  |  |  |
|  | Operating System: |  |  |  |
|  | - Microsoft Windows XP Professional (preferred) or Windows Vista Business - preinstalled and licensed, - CD and documentation included - including the latest service pack, patches and all critical updates - Serbian language user interface - Fully licensed recovery CD - All device drivers on CD |  |  |  |
|  | Microsoft Office Basic 2007 |  |  |  |
|  | - Preinstalled - Serbian language user interface |  |  |  |
|  | Adobe Acrobat Reader (latest version) |  |  |  |
|  | - Preinstalled |  |  |  |
| S_C_1.18 | Office Telephone |  |  |  |
|  | Features: |  |  |  |
|  | - Analogue PSTN telephone |  |  |  |
|  | - CLIP Support |  |  |  |
|  | - Alphanumeric LCD Display |  |  |  |
|  | - Full-Duplex hand-free function |  |  |  |
|  | - Redial button |  |  |  |
|  | - Adjustable receiver volume |  |  |  |
|  | - Adjustable ringtones |  |  |  |
|  | - Adjustable ringtone volume |  |  |  |
|  | - Integrated phone book |  |  |  |
|  | - Min. 9 speed dials |  |  |  |
|  | - RJ11 input |  |  |  |
|  | Accessories: |  |  |  |
|  | - RJ11 cable |  |  |  |
| S_C_1.19 | Office Furniture |  |  |  |
|  | Sideboard 90x45x62 (1 unit) Sideboard to use for IT equipment such as CPU, printer, scanner. Size: 90x45x62 cm. Material: fine chipboard, quality E1. |  |  |  |
|  | High Cabinet 105x40x190 (3 units) High cabinet with two revolving doors, Size: 105 x 40 x 190 cm. Design: side panels, base panel and doors made of fine chipboard, 20mm thick (E1). Visible surfaces: beech veneer (or melamine resin coated). |  |  |  |
|  | Coat rack (4 units) Freestanding coat rack with rounded footplate, overall dimensions H190x d56 cm. Minimum 8 coat hinges on hat crown. Material: beech, bent wood. Colour of the wooden parts matching the room furniture. |  |  |  |
|  | Bookshelves 30x60x230 (3 units) Freestanding heavy duty shelves modules. Size: 30 x 60 x 230cm. Side boards of fine chipboard 16mm thick, shelves of fine chipboard 19mm thick, quality E1. Visible surface: beech veneer or melamine resin coated with plastic edging-strip thickness 2mm. |  |  |  |
|  | Bookshelves 30x120x230 (3 units) Freestanding heavy duty shelves modules. Size: 30 x 60 x 230cm. Side boards of fine chipboard 16mm thick, shelves of fine chipboard 19mm thick, quality E1. Visible surface: beech veneer or melamine resin coated with plastic edging-strip thickness 2mm. |  |  |  |
|  | Office Desk Banana-shaped (6 units) Office desk and return will be supported by a beam based desk system. Minimum Dimensions: 180x92x72 cm. The worktops will be linked to a metal beam and this is supported by cold rolled steel plate shoulder or legs with set screws. Beam and legs will be provided with full wipe-in cable management. Static stability of the table frames - supporting metal profile, rigid construction. Desk top: fine chipboard 25mm thick, quality E1, low formaldehyde content. Frames: Steel with epoxy or polyester film painting shall be made by electrostatic process and shall offer an excellent resistance to scratches, friction and rust, integrated equipment for electrical installations, colour: selected by the client. |  |  |  |
|  | Multipurpose Table 160x80 (1 unit) Multipurpose table supported by strong metal frame and wooden top finishing. Height 720 mm, dimensions 1600x800 mm. Painting of the metallic parts with epoxy powder coat or polyester film resistant to scratches, friction and rust. Table plate of E1 chipboard low formaldehyde content 25mm thick and wooden finish in beech veneer. Static stability of the table frames - supporting metal profile, rigid construction. |  |  |  |
|  | Roll container with drawers 43x59x60 (6 units) Roll container with drawers made of plates E1 chipboard 19mm with melamine resin coated or wooden finish in beech veneer. Dimensions W430xH590xD600 mm. Top: beech veneer. Edge: beech veneer 3mm. Corpus: beech veneer. Front: beech veneer. Minimum one drawer lockable with ball bearing guides. To use in combination with office desk Banana-shaped. |  |  |  |
|  | Chair without armrest (6 units) Staple chair, seat and back upholstered. Quality and design: see stacking chair for visitors and offices. |  |  |  |
|  | Swivel chair with arms (6 units) Swivel chair with arms for use in the offices. Frame: aluminium, burnished frame with five feet. All supporting elements made of steel. Upholstered with adequate fabric, 100% crimp line CS,B1, Q1, min.380g/running metre, abrasion resistant with a minimum of 40,000 tours according to Martindale. |  |  |  |
| S_C_1.20 | Training Centre Furniture |  |  |  |
|  | High Cabinet 105x40x190 (1 unit) High cabinet with two revolving doors, Size: 105 x 40 x 190 cm. Design: side panels, base panel and doors made of fine chipboard, 20mm thick (E1). Visible surfaces: beech veneer (or melamine resin coated). |  |  |  |
|  | Coat rack (2 units) Freestanding coat rack with rounded footplate, overall dimensions H190x d56 cm. Minimum 8 coat hinges on hat crown. Material: beech, bent wood. Colour of the wooden parts matching the room furniture. |  |  |  |
|  | Bookshelves 30x60x230 (1 unit) Freestanding heavy duty shelves modules. Size: 30 x 60 x 230cm. Side boards of fine chipboard 16mm thick, shelves of fine chipboard 19mm thick, quality E1. Visible surface: beech veneer or melamine resin coated with plastic edging-strip thickness 2mm. |  |  |  |
|  | Bookshelves 30x120x230 (1 unit) Freestanding heavy duty shelves modules. Size: 30 x 60 x 230cm. Side boards of fine chipboard 16mm thick, shelves of fine chipboard 19mm thick, quality E1. Visible surface: beech veneer or melamine resin coated with plastic edging-strip thickness 2mm. |  |  |  |
|  | Office Desk Banana-shaped (7 units) Office desk and return will be supported by a beam based desk system. Minimum Dimensions: 180x92x72 cm. The worktops will be linked to a metal beam and this is supported by cold rolled steel plate shoulder or legs with set screws. Beam and legs will be provided with full wipe-in cable management. Static stability of the table frames - supporting metal profile, rigid construction. Desk top: fine chipboard 25mm thick, quality E1, low formaldehyde content. Frames: Steel with epoxy or polyester film painting shall be made by electrostatic process and shall offer an excellent resistance to scratches, friction and rust, integrated equipment for electrical installations, colour: selected by the client. |  |  |  |
|  | Roll container with drawers 43x59x60 (7 units) Roll container with drawers made of plates E1 chipboard 19mm with melamine resin coated or wooden finish in beech veneer. Dimensions W430xH590xD600 mm. Top: beech veneer. Edge: beech veneer 3mm. Corpus: beech veneer. Front: beech veneer. Minimum one drawer lockable with ball bearing guides. To use in combination with office desk Banana-shaped. |  |  |  |
|  | Swivel chair with arms (7 units) Swivel chair with arms for use in the offices. Frame: aluminium, burnished frame with five feet. All supporting elements made of steel. Upholstered with adequate fabric, 100% crimp line CS,B1, Q1, min.380g/running metre, abrasion resistant with a minimum of 40,000 tours according to Martindale. |  |  |  |

Land User Work Stations

| 1. Item Number | 2. Specifications | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| S_LWS_1.1 | Office PCs – LUWS Type 1 (34 units) |  |  |  |
|  | Features: |  |  |  |
|  | - Minitower Case |  |  |  |
|  | - Quad Core CPU, Min. 4x2,2 GHz |  |  |  |
|  | - Min. 2 GB DDR2-800 RAM (expandable up to 4 GB) |  |  |  |
|  | - Min. 2x300 GB HDD (SATA Raid Level 1) |  |  |  |
|  | - internal DVD±R DL, CD±RW |  |  |  |
|  | - 10/100/1000 MBit Ethernet adapter |  |  |  |
|  | - Min. 4 x USB v2.0 ports (min. 1 Front USB) |  |  |  |
|  | - Keyboard (USB or PS/2) |  |  |  |
|  | - Optical Mouse with scroll wheel (USB or PS/2) |  |  |  |
|  | - Dual Head graphics adapter min. 256MB RAM, DVI interface |  |  |  |
|  | Accessories: |  |  |  |
|  | - Cat.5e patch cable, flexible, sealed connectors, length 5m |  |  |  |
|  | - Power cord |  |  |  |
|  | Operating System: |  |  |  |
|  | Microsoft Windows XP Professional (preferred) or Windows Vista business – preinstalled and licensed, CD and documentation included – including the latest service pack, patches and all critical updates – Serbian language user interface – Fully licensed recovery CD – All device drivers on CD |  |  |  |
|  | Software: |  |  |  |
|  | Microsoft Office Basic 2007 – preinstalled – Serbian language user interface Adobe Acrobat Reader (latest version) |  |  |  |
| S_LWS_1.2 | TFT LCD screens (68 units) |  |  |  |
|  | Features: |  |  |  |
|  | - min. 22” screen size |  |  |  |
|  | - min. resolution 1280x1024 pixel |  |  |  |
|  | - reaction time <10ms |  |  |  |
|  | - contrast > 700:1 |  |  |  |
|  | - brightness > 250cd/m² |  |  |  |
|  | - viewing angle >= 160/160 |  |  |  |
|  | - integrated loudspeakers |  |  |  |
|  | - DVI video input |  |  |  |
|  | Accessories: |  |  |  |
|  | - DVI video cable, shielded, min. 2 m length |  |  |  |
|  | - Power cord |  |  |  |
|  | - Audio cable, min 2m length |  |  |  |
| S_LWS_1.3 | A4 colour laser printers (32 units) |  |  |  |
|  | Technology: |  |  |  |
|  | - Laser colour, A4 format - Resolution at least 600x600 dpi - At least 10 pages per minute print speed (monochrome) - At least 64 MB RAM - Min. 1 tray with minimum 150 sheets |  |  |  |
|  | Interface: |  |  |  |
|  | - USB 2.0 PC connection port - Microsoft Windows XP and Vista drivers |  |  |  |
|  | Accessories: |  |  |  |
|  | - Power cord - USB connection cable to PC - Two toner cartridges per colour |  |  |  |
|  | Warranty: |  |  |  |
|  | - One (1) year on-site warranty |  |  |  |
| S_LWS_1.4 | Uninterrupted power supply (UPS) – (34 units) |  |  |  |
|  | Output power capacity: |  |  |  |
|  | - Output power capacity: min. 2000VA/1200W |  |  |  |
|  | - Nominal output voltage: 230 VAC |  |  |  |
|  | - Nominal input voltage: 230 VAC |  |  |  |
|  | - Output frequency: 50 Hz |  |  |  |
|  | - Input frequency: 50 Hz |  |  |  |
|  | Interface: |  |  |  |
|  | - Ethernet SNMP interface - Microsoft Windows XP and vista drivers (if necessary) |  |  |  |
|  | Software: |  |  |  |
|  | - Software for automatic shut-down and auto restart of the protected computer systems. - Software that enables remote monitoring of the UPS - Local and remote shutdown - Power management software for UPS for Windows XP/Vista, CD and documentation included - Microsoft Windows XP/Vista drivers |  |  |  |
|  | Interface: |  |  |  |
|  | - Power cord - Cat.5e patch cable, flexible, sealed connectors, length 5m |  |  |  |
|  | Acoustic noise |  |  |  |
|  | - Max. 40 dB |  |  |  |
|  | Warranty |  |  |  |
|  | - One (1) year on-site warranty |  |  |  |
| S_LWS_1.5 | Mobile Land User Work Stations – LUWS type 2 (7 units) |  |  |  |
|  | Features: |  |  |  |
|  | - Min. 14,1” screen, non-glare |  |  |  |
|  | - Min. resolution 1280x800 pixel |  |  |  |
|  | - Min Dual Core CPU, 2x2 GHz |  |  |  |
|  | - 2 GB DDR2 RAM (expandable up to 4 GB) |  |  |  |
|  | - Min 160 GB HDD |  |  |  |
|  | - internal DVD±R DL, CD±RW |  |  |  |
|  | - 10/100/1000 MBit Ethernet adapter |  |  |  |
|  | - Integrated wireless LAN 802.11 b/g/n |  |  |  |
|  | - Min 4 x USB 2.0 ports |  |  |  |
|  | - PCMCIA or PC Express card slot |  |  |  |
|  | - VGA or DVI out |  |  |  |
|  | - Integrated 3 in 1 card reader |  |  |  |
|  | - Li-Ion battery for min. 2 hours of independent operation |  |  |  |
|  | - Integrated track pad |  |  |  |
|  | Accessories: |  |  |  |
|  | - Cat.5e patch cable, flexible, sealed connectors, length 5m |  |  |  |
|  | - AC adapter with power cord |  |  |  |
|  | - Optical mouse with scroll wheel (USB) |  |  |  |
|  | - Carrying case |  |  |  |
|  | Operating System: |  |  |  |
|  | - Microsoft Windows XP Professional (preferred) or Windows Vista Business - preinstalled and licensed, - CD and documentation included - including the latest service pack, patches and all critical updates - Serbian language user interface (3 units), English language user interface (4 units) - Fully licensed recovery CD - All device drivers on CD |  |  |  |
|  | Microsoft Office Basic 2007 |  |  |  |
|  | - Preinstalled - Serbian language user interface (3 units), English language user interface (4 units) |  |  |  |
|  | Adobe Acrobat Reader (latest version) |  |  |  |
|  | - Preinstalled |  |  |  |

Lot 2: RIS-specific equipment

Tracking & Tracing System

| 1. Item Number | 2. Specifications | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| S_TT_1.1 | Standard AIS Base Station type 1 |  |  |  |
|  | Base Station Transponder: |  |  |  |
|  | - 230 VAC input power |  |  |  |
|  | - Receiver sensitivity >-107dBm |  |  |  |
|  | - MTBF >= 40.000h |  |  |  |
|  | - GPS Antenna |  |  |  |
|  | - Operating temperature -15 to +55°C |  |  |  |
|  | - Approval for IEC 62320-1 |  |  |  |
|  | - Notification of radio equipment for the use in Serbia |  |  |  |
|  | - Applies to IEC 62320-1 |  |  |  |
|  | - Applies to IEC 61993-2 |  |  |  |
|  | - Applies to IEC 60950 |  |  |  |
|  | - Applies to ITU-R M. 1371 |  |  |  |
|  | - Applies to IALA A-124 |  |  |  |
|  | Industrial PC: |  |  |  |
|  | - 19” rack mount case max. 4 height units or other rack mount option |  |  |  |
|  | - Min 1 GHz Low Voltage Processor with passive cooling |  |  |  |
|  | - Min. 2 GB RAM |  |  |  |
|  | - Non HDD drive (Solid State, Flash, etc.) min. 80GB |  |  |  |
|  | - Min. 4 x 10/100/1000 MBit Ethernet adapter |  |  |  |
|  | - Min.3 x USB v2.0 ports (min. 1 Front USB) |  |  |  |
|  | - Min. 1 RS 232 serial interface |  |  |  |
|  | - Operating temperature -15 to +55°C |  |  |  |
|  | - MTBF >= 100.000h |  |  |  |
|  | UPS |  |  |  |
|  | - Output power capacity: min. 3000VA |  |  |  |
|  | - Optional Battery Pack (if necessary) |  |  |  |
|  | - Nominal output voltage: 230 VAC |  |  |  |
|  | - Nominal input voltage: 230 VAC |  |  |  |
|  | - Output frequency: 50 Hz |  |  |  |
|  | - Input frequency: 50 Hz |  |  |  |
|  | - Standard output for backup power: min. 4 IEC 320 C13 outlets |  |  |  |
|  | - Two separate controllable power segments |  |  |  |
|  | - Automatic internal bypass |  |  |  |
|  | - Maintenance free batteries |  |  |  |
|  | - Automatic self test |  |  |  |
|  | - Cold start capability |  |  |  |
|  | - Ethernet SNMP interface |  |  |  |
|  | - Drivers for Windows and Linux OS |  |  |  |
|  | - Software for automatic shut-down and auto restart of the protected computer systems |  |  |  |
|  | - Software that enables remote monitoring of the UPS |  |  |  |
|  | - Input power cord |  |  |  |
|  | - 2 IEC 320 C13 power cords for external components |  |  |  |
|  | - 2 power extension units, 19” rack mount with min. 4 sockets each |  |  |  |
|  | - Cat.5e patch cable, flexible, sealed connectors, length 5m |  |  |  |
|  | - One year on site warranty |  |  |  |
|  | KVM TFT unit |  |  |  |
|  | - 19” rack mount |  |  |  |
|  | - Max. 2 height units folded |  |  |  |
|  | - Min. 15” TFT display |  |  |  |
|  | - Keyboard plus trackball |  |  |  |
|  | - Min. 4 KVM ports |  |  |  |
|  | Network switch |  |  |  |
|  | - 19” rack mount |  |  |  |
|  | - 10/100/1000MBit |  |  |  |
|  | - Min. 8 ports |  |  |  |
|  | 19” rack for housing of BS equipment |  |  |  |
|  | - Enclosure frame made of stainless steel |  |  |  |
|  | - Aluminium side panels left and right, powder coated |  |  |  |
|  | - Aluminium front and back door, powder coated |  |  |  |
|  | - Aluminium roof, powder coated |  |  |  |
|  | - Min. 40 height units |  |  |  |
|  | - Min. depth 650mm |  |  |  |
|  | - Earthing kit |  |  |  |
|  | - All necessary rails, screws and other mounting equipment for installation of BS components |  |  |  |
|  | Installation: |  |  |  |
|  | - Delivery pre-assembled in 19” rack and pre-configured to a location in Belgrade |  |  |  |
| S_TT_1.2 | Standard AIS Base Station type 2 |  |  |  |
|  | Base Station Transponder: |  |  |  |
|  | - 230 VAC input power |  |  |  |
|  | - Receiver sensitivity >-107dBm |  |  |  |
|  | - MTBF >= 40.000h |  |  |  |
|  | - GPS Antenna |  |  |  |
|  | - Operating temperature -15 to +55°C |  |  |  |
|  | - Approval for IEC 62320-1 |  |  |  |
|  | - Notification of radio equipment for the use in Serbia |  |  |  |
|  | - Applies to IEC 62320-1 |  |  |  |
|  | - Applies to IEC 61993-2 |  |  |  |
|  | - Applies to IEC 60950 |  |  |  |
|  | - Applies to ITU-R M. 1371 |  |  |  |
|  | - Applies to IALA A-124 |  |  |  |
|  | Industrial PC: |  |  |  |
|  | - 19” rack mount case max. 4 height units or other rack mount option |  |  |  |
|  | - Min 1 GHz Low Voltage Processor with passive cooling |  |  |  |
|  | - Min. 2 GB RAM |  |  |  |
|  | - Non HDD drive (Solid State, Flash, etc.) min. 80GB |  |  |  |
|  | - Min. 4 x 10/100/1000 MBit Ethernet adapter |  |  |  |
|  | - Min.3 x USB v2.0 ports (min. 1 Front USB) |  |  |  |
|  | - Min. 1 RS 232 serial interface |  |  |  |
|  | - Operating temperature -15 to +55°C |  |  |  |
|  | - MTBF >= 100.000h |  |  |  |
|  | UPS |  |  |  |
|  | - Output power capacity: min. 6000VA |  |  |  |
|  | - Optional Battery pack (if necessary) |  |  |  |
|  | - Nominal output voltage: 230 VAC |  |  |  |
|  | - Nominal input voltage: 230 VAC |  |  |  |
|  | - Output frequency: 50 Hz |  |  |  |
|  | - Input frequency: 50 Hz |  |  |  |
|  | - Standard output for backup power: min. 4 IEC 320 C13 outlets |  |  |  |
|  | - Two separate controllable power segments |  |  |  |
|  | - Automatic internal bypass |  |  |  |
|  | - Maintenance free batteries |  |  |  |
|  | - Automatic self test |  |  |  |
|  | - Cold start capability |  |  |  |
|  | - Ethernet SNMP interface |  |  |  |
|  | - Drivers for Windows and Linux OS |  |  |  |
|  | - Software for automatic shut-down and auto restart of the protected computer systems |  |  |  |
|  | - Software that enables remote monitoring of the UPS |  |  |  |
|  | - Power cord |  |  |  |
|  | - 2 IEC 320 C13 power cords for external components |  |  |  |
|  | - 2 power extension units, 19” rack mount with min. 4 sockets each |  |  |  |
|  | - Cat.5e patch cable, flexible, sealed connectors, length 5m |  |  |  |
|  | - One year on site warranty |  |  |  |
|  | KVM TFT unit |  |  |  |
|  | - 19” rack mount |  |  |  |
|  | - Max. 2 height units folded |  |  |  |
|  | - Min. 15” TFT display |  |  |  |
|  | - Keyboard plus trackball |  |  |  |
|  | - Min. 4 KVM ports |  |  |  |
|  | Network switch |  |  |  |
|  | - 19” rack mount |  |  |  |
|  | - 10/100/1000MBit |  |  |  |
|  | - Min. 8 ports |  |  |  |
|  | 19” rack for housing of BS equipment |  |  |  |
|  | - Enclosure frame made of stainless steel |  |  |  |
|  | - Aluminium side panels left and right, powder coated |  |  |  |
|  | - Aluminium front and back door, powder coated |  |  |  |
|  | - Aluminium roof, powder coated |  |  |  |
|  | - Min. 40 height units |  |  |  |
|  | - Min. depth 650mm |  |  |  |
|  | - Earthing kit |  |  |  |
|  | - All necessary rails, screws and other mounting equipment for installation of BS components |  |  |  |
|  | Installation: |  |  |  |
|  | - Delivery pre-assembled in 19” rack and pre-configured to a location in Belgrade |  |  |  |
| S_TT_1.3 | Standard AIS Base Station type 3 |  |  |  |
|  | Redundant Base Station Transponder (2 pieces): |  |  |  |
|  | - 230 VAC input power |  |  |  |
|  | - Receiver sensitivity >-107dBm |  |  |  |
|  | - MTBF >= 40.000h |  |  |  |
|  | - GPS Antenna |  |  |  |
|  | - Operating temperature -15 to +55°C |  |  |  |
|  | - Approval for IEC 62320-1 |  |  |  |
|  | - Notification of radio equipment for the use in Serbia |  |  |  |
|  | - Applies to IEC 62320-1 |  |  |  |
|  | - Applies to IEC 61993-2 |  |  |  |
|  | - Applies to IEC 60950 |  |  |  |
|  | - Applies to ITU-R M. 1371 |  |  |  |
|  | - Applies to IALA A-124 |  |  |  |
|  | Redundant Industrial PC (2 pieces): |  |  |  |
|  | - 19” rack mount case max. 4 height units or other rack mount option |  |  |  |
|  | - Min 1 GHz Low Voltage Processor with passive cooling |  |  |  |
|  | - Min. 2 GB RAM |  |  |  |
|  | - Non HDD drive (Solid State, Flash, etc.) min. 80GB |  |  |  |
|  | - Min. 4 x 10/100/1000 MBit Ethernet adapter |  |  |  |
|  | - Min.3 x USB v2.0 ports (min. 1 Front USB) |  |  |  |
|  | - Min. 1 RS 232 serial interface |  |  |  |
|  | - Operating temperature -15 to +55°C |  |  |  |
|  | - MTBF >= 100.000h |  |  |  |
|  | Antenna Control Unit: |  |  |  |
|  | - for connection of second AIS BS Transponder |  |  |  |
|  | UPS |  |  |  |
|  | - Output power capacity: min. 8000VA |  |  |  |
|  | - Optional Battery pack (if necessary) |  |  |  |
|  | - Nominal output voltage: 230 VAC |  |  |  |
|  | - Nominal input voltage: 230 VAC |  |  |  |
|  | - Output frequency: 50 Hz |  |  |  |
|  | - Input frequency: 50 Hz |  |  |  |
|  | - Standard output for backup power: min. 4 IEC 320 C13 outlets |  |  |  |
|  | - Two separate controllable power segments |  |  |  |
|  | - Automatic internal bypass |  |  |  |
|  | - Maintenance free batteries |  |  |  |
|  | - Automatic self test |  |  |  |
|  | - Cold start capability |  |  |  |
|  | - Ethernet SNMP interface |  |  |  |
|  | - Drivers for Windows and Linux OS |  |  |  |
|  | - Software for automatic shut-down and auto restart of the protected computer systems |  |  |  |
|  | - Software that enables remote monitoring of the UPS |  |  |  |
|  | - Power cord |  |  |  |
|  | - 2 IEC 320 C13 power cords for external components |  |  |  |
|  | - 2 power extension units, 19” rack mount with min. 4 sockets each |  |  |  |
|  | - Cat.5e patch cable, flexible, sealed connectors, length 5m |  |  |  |
|  | - One year on site warranty |  |  |  |
|  | KVM TFT unit |  |  |  |
|  | - 19” rack mount |  |  |  |
|  | - Max. 2 height units folded |  |  |  |
|  | - Min. 15” TFT display |  |  |  |
|  | - Keyboard plus trackball |  |  |  |
|  | - Min. 4 KVM ports |  |  |  |
|  | Network switch |  |  |  |
|  | - 19” rack mount |  |  |  |
|  | - 10/100/1000MBit |  |  |  |
|  | - Min. 8 ports |  |  |  |
|  | 19” outdoor rack for housing of BS equipment |  |  |  |
|  | - Enclosure frame made of stainless steel |  |  |  |
|  | - Aluminium side panels left and right, powder coated |  |  |  |
|  | - Aluminium front and back door, powder coated |  |  |  |
|  | - Aluminium roof, powder coated |  |  |  |
|  | - Min. 40 height units |  |  |  |
|  | - Min. depth 650mm |  |  |  |
|  | - 1000W Air-condition |  |  |  |
|  | - IP 55 protection |  |  |  |
|  | - Earthing kit |  |  |  |
|  | - All necessary rails, screws and other mounting equipment for installation of BS components |  |  |  |
|  | Installation: |  |  |  |
|  | - Delivery pre-assembled in 19” rack and pre-configured to a location in Belgrade |  |  |  |
| S_TT_1.4 | Directed VHF antennas |  |  |  |
|  | Features: |  |  |  |
|  | - Min. 4 element Yagi antenna |  |  |  |
|  | - Operating frequency 162 MHz |  |  |  |
|  | - Input impedance 50 ohm |  |  |  |
|  | - Gain min. 7 dBd |  |  |  |
|  | - Front to Back ration >17dB |  |  |  |
|  | - VSWR < 2:1 |  |  |  |
|  | - Connector type N female |  |  |  |
|  | - Wind resistance min. 160 km/h |  |  |  |
|  | - Operation temperature -35 to +80°C |  |  |  |
| S_TT_1.5 | Omni VHF antennas |  |  |  |
|  | Features: |  |  |  |
|  | - Operating frequency 162 MHz |  |  |  |
|  | - Input impedance 50 ohm |  |  |  |
|  | - Gain min. 4 dBd |  |  |  |
|  | - VSWR < 1,5:1 |  |  |  |
|  | - Connector type N female |  |  |  |
|  | - Wind resistance min. 160 km/h |  |  |  |
|  | - Operation temperature -35 to +80°C |  |  |  |
| S_TT_1.6 | Two-way antenna coupler |  |  |  |
|  | Features: |  |  |  |
|  | - Operating frequency 162 MHz |  |  |  |
|  | - Input impedance 50 ohm |  |  |  |
|  | - Insertion loss < 0,15 dB |  |  |  |
|  | - VSWR < 1,2:1 |  |  |  |
|  | - Connector type 3xN female |  |  |  |
|  | - Operation temperature -35 to +80°C |  |  |  |
| S_TT_1.7 | VHF cable |  |  |  |
|  | Features: |  |  |  |
|  | - Attenuation / 100m < 4dB @ 160 MHz |  |  |  |
|  | - Length 450m |  |  |  |
|  | Accessories: |  |  |  |
|  | - Grounding clamp (20 pieces) |  |  |  |
|  | - Connector type N male (100 pieces) |  |  |  |
| S_TT_1.8 | GPS signal splitter |  |  |  |
|  | Features: |  |  |  |
|  | - 1 input |  |  |  |
|  | - 4 outputs |  |  |  |
|  | - TNC connectors |  |  |  |
|  | - Active amplification |  |  |  |
|  | - DC blocking |  |  |  |
|  | Accessories: |  |  |  |
|  | - Fitting power supply unit for 230 VAC input voltage |  |  |  |
|  | - Mounting kit |  |  |  |
| S_TT_1.9 | AIS DGPS data generation module |  |  |  |
|  | Features: |  |  |  |
|  | - Min. GPS L1 band receiver |  |  |  |
|  | - Min. 24 channels GPS and 2 SBAS |  |  |  |
|  | - Adjustable position and raw-data output up to 20 Hz |  |  |  |
|  | - Differential base or remote RTCM SC 104 V2.3 output |  |  |  |
|  | - Standard NMEA-0183 V3.01 output |  |  |  |
|  | - Min. 3 serial (RS 232) ports |  |  |  |
|  | - Min. 1 USB port |  |  |  |
|  | - Position accuracy autonomous (CEP) <= 2m |  |  |  |
|  | - Position accuracy local DGPS (CEP) <= 0,5m |  |  |  |
|  | - Position accuracy SBAS (CEP) <= 1m |  |  |  |
|  | - TNC connectors |  |  |  |
|  | - 5V GPS antenna compatibility |  |  |  |
|  | Accessories: |  |  |  |
|  | - Suitable power supply for 230 VAC input voltage |  |  |  |
|  | - Connection cable 50Ω, 2xTNC male, min. 3m length |  |  |  |

ENC Production and Distribution System

| 1. Item Number | 2. Specifications | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| S_ENC_2.1 | Digital Ortho-photo images of entire Danube |  |  |  |
|  | Coverage area: |  |  |  |
|  | - Corridor along the Danube river between river-km 1433 and 846. - On each side of the river axis 2 km coverage |  |  |  |
|  | Data quality: |  |  |  |
|  | - Digital Ortho-photo images, geo-referenced in WGS84 no older than 5 years - Aerial ortho-photos are preferred - Where no aerial data is available, ortho-photos created from satellite images are accepted - Resolution: aerial ortho-photos: 50cm/pixel or better satellite ortho-photos: 1m/pixel or better |  |  |  |

D4D International Data Exchange System

| 1. Item Number | 2. Specifications | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| S_D4D_1.1 | Handheld GPS receivers |  |  |  |
|  | Features: - Display LCD - Battery life: 20 hours - Waterproof: yes - GPS Receiver: 12 channel, EGNOS, NMEA |  |  |  |
|  | Interfaces: - USB |  |  |  |

Land User Work Stations

| 1. Item Number | 2. Specifications | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| S_LWS_1.6 | VHF radio station |  |  |  |
|  | Features: |  |  |  |
|  | - Min. frequency range 146 – 174 MHz |  |  |  |
|  | - Min. 10 channels |  |  |  |
|  | - Output power 1-25 W |  |  |  |
|  | - Microphone with channel selector |  |  |  |
|  | - ATIS decoder (or external accessory) |  |  |  |
|  | - BNC antenna adapter |  |  |  |
|  | - Operation temperature range: -20° to +55°C |  |  |  |
|  | - IP 54 protected |  |  |  |
|  | - Conforms to RTTE directive 1999/5/EC |  |  |  |
|  | - Notified for the use as shore station in Serbia |  |  |  |
|  | Accessories: |  |  |  |
|  | External loudspeaker / microphone unit: - Desktop type - Directly connected to VHF radio station - Remote control of VHF radio station - LCD display - PTT button - Programmable channel buttons |  |  |  |
|  | ATIS decoder (if not included in the station) |  |  |  |

Vessel User Terminals

Technical Annex 1 for Vessel User Terminal (VUT_TA_1):

The Inland ECDIS viewer is responsible for display of ENCs according to the Inland ECDIS Standard and - when connected to an Inland AIS transponder - displays tactic traffic image onboard ships. It is also an input device for input and maintenance of Inland AIS related data into the Inland AIS transponder. It must provide the possibility to fully operate and configure the Inland AIS unit. Furthermore it provides the possibility to:

- Display ship static and voyage related data of the own ship.

- Input ship static and voyage related data of own ship and sending into the Inland AIS transponder.

- Display safety related/PoB messages and input them into the Inland AIS transponder.

- Input ETA messages into the transponder.

- Display received RTA messages from the transponder

- Textual display of received water level messages

The following requirements are mandatory for the Inland ECDIS Viewer Application.

The normal operation of the Inland ECDIS viewer has to be carried out by the use of a trackball.

The Inland ECDIS viewer shall be able to import ENC in the S-57 (.000) and 7CB format (version 1.2 and 2.0) easily via CD/DVD or USB stick. The error free handling of different layers is mandatory. The Inland ECDIS viewer shall display charts and objects according to the standard and shall allow pick reports on ECDIS objects as well as on AIS objects. Furthermore it should support the production of screenshots and the export of those screenshots to an external media (CD-R/DVD-R or USB stick).

The screen brightness and the ECDIS colour schemes of the ENC viewer application shall be adjustable for work under different light conditions in accordance with the Inland ECDIS standard. Also the display of different levels of detail of chart data shall easily be possible.

The language of the ECDIS Viewer Application shall be switchable between Serbian and English. All coded Inland AIS data which will be decoded by the ECDIS Viewer Application shall also follow the language set.

The software shall be delivered with the official charts for Serbia already installed on the system. Chart and software updates for one year, starting after the successful system acceptance shall be included in the offer. Chart and software update contracts for the time after the first year can be offered optionally.

In Information Mode all kinds of chart orientation, rotation, zooming and panning must be possible. Standard chart orientation shall be north up or orientation to the fairway axis at the actual position.

It should be possible to change the chart position by user actions like click-to-centre, scrolling along the fairway axis or by selection of a river kilometre as the new chart centre position.

The visualisation of AIS information shall provide the following functionalities:

- The symbol for the own ship shall not indicate any orientation if no heading information is available.

- A circle symbol shall be used for display of other AIS targets if no heading information is being transmitted.

- In case that heading information is being available, a triangle symbol shall be used for display of those AIS targets. Alternatively the user shall be able to switch to the display of ship outline according to dimension information from AIS if heading is available.

- The standard colour for the display of AIS symbols is black. If a ship carries dangerous cargo (according to AIS information: maritime ship type cargo indication 1-4 or inland number of blue cones > 0) the colour of symbol shall change to red.

- All ships having high position accuracy (according to AIS information) shall use solid lines for the symbols used. If ships have low position accuracy (according to AIS information) all symbols shall use dashed lines;

- If blue sign availability is indicated via AIS a blue square shall be displayed beside (on the right hand side in case of heading) the ship symbol. If the blue sign flag in the Inland AIS position report is being set the blue square shall be filled with blue colour.

- It shall be possible to optionally display certain AIS information (minimum MMSI number, name and velocity) directly attached to each AIS symbol.

- The display of AIS information next to the symbols shall be switchable for single targets, all targets or none targets.

- It shall be configurable if and for which period of time AIS past tracks (the reported AIS positions) shall be displayed for all vessels.

- It shall be possible to display the whole set of AIS data for selected ships in an own dialog window or screen section.

- It shall be possible to manually perform an AIS interrogation for (inland) ship static and voyage related data on selected ships.

The following functions regarding the interaction with the onboard Inland AIS unit are required:

- Reading and writing of the whole set of own ship AIS data to connected Inland AIS unit.
The Inland ECDIS viewer software must provide the capabilities to set all data items in the connected Inland AIS unit using the AIS related NMEA input sentences specified in IEC 61162 and in the Inland AIS standard. The Inland ECDIS viewer shall make automatic conversion of inland and maritime entries. This shall be implemented for vessel type and cargo class, as well as for ship draught. The menu shall provide a list of all inland specific vessel types and automatically set the maritime vessel type accordingly. Also when a ship draught is entered in centimetre accuracy it shall be calculated into a decimetre value (rounded upwards) for use in AIS message 5.

- The display of all Inland AIS data of AIS targets within the VHF coverage range of the Inland AIS transponder shall be possible by easy user action. An overview containing at least ship names shall allow the selection of a single ship for viewing the whole set of available Inland AIS data.

- Reception of safety related and PoB messages.
Safety related AIS messages and PoB messages addressed to the own ship and received by the Inland AIS transponder must result in a visible notification in the ECDIS Viewer Application. A single user action must then show at least the content, the sender and the reception time of the messages. Scrolling must be implemented when necessary.

- Sending of Safety Related Messages (SRM).
The ECDIS Viewer Application must provide the function to open a dialogue for sending of safety related and PoB messages. Within the dialogue it can be decided whether to send a broad or an addressed message. If an addressed message is chosen a list of all received AIS targets shall be presented for selection of the destination. Further a list of pre-defined messages according to the “Recommended expressions” chapter 3 of the “Standardized UN/ECE Vocabulary for Radio-Connections” must be selectable for sending. Other alternatives are the transmission of a Persons on Board message where the text “Man over Board at” shall be added by the actual Lat/Lon of given by the GPS of the Inland AIS transponder, or the possibility to enter a free text message.

- Sending of addressed Safety Related Messages.
In addition to the procedure described before, the selection of a target vessel for the transmission of a SRM or PoB message must also be possible by user interaction with the desired ship symbol on the ECDIS display (e.g. by showing a menu when clicking on the ship with a mouse button). In that case the previously described menu shall show up with type and destination of the message already selected.

- Sending of ETA messages.
The ECDIS Viewer Application must provide the function to open a dialogue for sending of an ETA message. Therefore it must store a list of virtual MMSI numbers for different countries as well as an RIS Index. Both lists must be locally stored and must be editable by the user. When an ETA message shall be sent, the input of the necessary data must be supported by pull down menus for destination MMSI and location codes. For the entry of the location code, the application must filter the full RIS index according to the already selected values e.g. if the country code has been chosen, only location codes of this country must be available in the following menus. The RIS index and the list of virtual MMSIs have to be provided by the client. The successful tenderer has to specify the required format for import of the RIS index. At least an import of a .csv format has to be supported.

- Reception of RTA messages.
RTA messages addressed to the own ship and received by the Inland AIS transponder must result in a visible notification in the ECDIS Viewer Application. A single user action must then show the content, the sender and the reception time of the messages. Scrolling must be implemented when necessary.

- Reception of water level messages and textual display via menu.
Water level messages received by the Inland AIS transponder must be accessible in the Inland ECDIS Viewer Application either by selection via a menu or by continuous display on the main screen of the application.

All components have to comply with the environmental conditions on board of vessels.

Technical Annex 2 for Vessel User Terminal (VUT_TA_2):

In Navigation Mode, the Inland ECDIS display shall be integrated with the own ship's radar information. The radar information shall be clearly distinguishable from the SENC information and the radar image shall have the highest display priority.

The under laid SENC shall match in position, range and orientation. The radar image and the position from the position sensor shall both be adjustable for the antenna offset to the conning position.

It shall be possible to remove either the ECDIS or the radar information by a single operator action temporarily. The ECDIS information shall be removable by user action also in the long term. The display of Tracking & Tracing information from Inland AIS must also be removable by user action in the long term.

The Inland ECIDS viewer in Navigation mode must provide an indication when the input from the position-fixing system is lost. It shall also repeat, but only as an indication, any alarm or indication passed to it from a position fixing system.

The positioning system and the SENC shall be based on the same geodetic datum.

Information regarding the position and orientation of other vessels, gathered by other communication links than the own radar, are permitted to be displayed only if they are up-to-date (nearly real-time) and meet the accuracy that is required for the support of tactical and operational navigation.

Any Inland AIS symbols shall not disturb the radar image during passing and shall be faded out therefore if the user decides so. The application should then allow the skipper to define the area where the Inland AIS symbols are faded out.

The Application shall be able to change to Information mode on user action. It shall be clearly and continuously indicated that the application is then running in Information mode.

| 1. Item Number | 2. Specifications | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| S_VUT_1.1 | Inland AIS mobile station |  |  |  |
|  | Features: |  |  |  |
|  | - Input power 18 – 36 VDC - Receiver sensitivity <-107dBm - Full integration of ETA/RTA functionality into Inland AIS transponder unit - Full integration of PoB functionality into Inland AIS transponder unit - Full integration of water level functionality into Inland AIS transponder unit |  |  |  |
|  | Components: |  |  |  |
|  | - Inland AIS transponder unit - Inland AIS transponder unit mounting accessories - Power supply cable for Inland AIS unit, 10 m length (fused) - External power switch for Inland AIS unit - DC-DC converter (optional) - MKD (integrated or external) - MKD mounting accessories (if external MKD) - Power supply cable for MKD, 10 m length (if needed) - Interface cable from Inland AIS transponder unit to MKD, 5m length (if external MKD) |  |  |  |
|  | Accessories: |  |  |  |
|  | - VHF Omni directional antenna - Mounting equipment for VHF antenna - VHF cable for VHF antenna, 20 m length (with suitable attenuation) - GPS antenna - Mounting equipment for GPS antenna - GPS antenna cable for GPS antenna, 15 m length (with suitable attenuation) - USB interface adaptor for connection of an Inland ECDIS viewer - USB cable (USB A male end connector) for connection of Inland ECDIS viewer, 5 m length - One sensor interface cable, 10 m length - All other necessary parts and components - Operation manual (Serbian (preferred) or English language) - Installation manual (Serbian (preferred) or English language) |  |  |  |
|  | Environmental conditions: |  |  |  |
|  | - operating temperature -15 to +55°C |  |  |  |
|  | Required approvals: |  |  |  |
|  | - EC type examination certificate (Wheelmark certificate) from BSH Hamburg - Inland AIS certificate from FVT Koblenz - Notification for the use in Serbia |  |  |  |
|  | Applicable Standards: |  |  |  |
|  | - IMO MSC. 74(69) Annex 3 - ITU-R M. 1371 - ITU-R M. 825-3 - ITU-R M. 1084-3 - IEC 61993-2 - IEC 61162-1 - IEC 61162-2 - IEC 61108-1 - IALA Technical Clarifications of ITU-R M. 1371-1 - IEC 60945 - COMMISSION REGULATION (EC) No 415/2007 (Inland AIS) |  |  |  |
| S_VUT_1.2 | Portable Inland AIS mobile station |  |  |  |
|  | Features: |  |  |  |
|  | - Standard Inland AIS transponder with MKD - Mounted in water proof case (IP 67, floatable) - no modifications of the Inland AIS components allowed, it must be possible to extract the unit at a later time - Isolated connectors for VHF and GPS antenna, DC power supply and PI - Input power 18 – 36 VDC - Receiver sensitivity <-107dBm |  |  |  |
|  | Components: |  |  |  |
|  | - Water proof case incl. assembling material for AIS and MKD - Inland AIS transponder unit - Power supply cable for Inland AIS unit, with connector to case outlet,10 m length (fused) - DC-DC converter (optional) - MKD (integrated or external) - Power supply cable for MKD (if needed) - Interface cable from Inland AIS transponder unit to MKD, (if external MKD) |  |  |  |
|  | Accessories: |  |  |  |
|  | - VHF Omni directional antenna - Mounting equipment for VHF antenna - VHF cable for VHF antenna with connector for case outlet, 20 m length (with suitable attenuation) - GPS antenna - Mounting equipment for GPS antenna - GPS antenna cable for GPS antenna with connector for case outlet, 15 m length (with suitable attenuation) - USB interface adaptor for connection of an Inland ECDIS viewer - USB cable with connector for case outlet (USB A male end connector) for connection of Inland ECDIS viewer, 5 m length - One sensor interface cable, 10 m length - All other necessary parts and components - Operation manual (Serbian (preferred) or English language) - Installation manual (Serbian (preferred) or English language) |  |  |  |
|  | Environmental conditions: |  |  |  |
|  | - operating temperature -15 to +55°C - IP 67 |  |  |  |
|  | Required approvals: |  |  |  |
|  | - EC type examination certificate (Wheelmark certificate) from BSH Hamburg - Inland AIS certificate from FVT Koblenz - Notification for the use in Serbia |  |  |  |
|  | Applicable Standards: |  |  |  |
|  | - IMO MSC. 74(69) Annex 3 - ITU-R M. 1371 - ITU-R M. 825-3 - ITU-R M. 1084-3 - IEC 61993-2 - IEC 61162-1 - IEC 61162-2 - IEC 61108-1 - IALA Technical Clarifications of ITU-R M. 1371-1 - IEC 60945 - COMMISSION REGULATION (EC) No 415/2007 (Inland AIS) |  |  |  |
| S_VUT_1.3 | Digital Radar System |  |  |  |
|  | Features: |  |  |  |
|  | - Frequency range 9300 – 9500 MHz - CE conformity according to RTT 1999/5/EG - Notification for the use in Serbia - Antenna length min. 200cm, max. 230m - Digital interface for coupling with Inland ECDIS device - CCNR approval |  |  |  |
|  | Components: |  |  |  |
|  | - Radar Antenna - Radar Processor - Radar control board - Daughter interface card for connection of Inland ECDIS Software in Navigation mode (compatible to S_VUT_1.3c) |  |  |  |
| S_VUT_1.4 | Digital Heading device |  |  |  |
|  | Features: |  |  |  |
|  | - Conforms to IMO resolution MSC.116(3) - Static heading error max.. ± 1° - Dynamic heading error max. ± 1,5° - Rate of Turn error max. 1°/s - Configurable output sentences - Min. output sentences: HDT, ROT, GGA - Maximum output rate min. 20 Hz - Min. 1 RS 232 output - Min. 1 RS 422 output - Supply voltage range min.12-24 VDC - Operation temperature of sensor unit: min. -25° to +55° C - Environmental protection of sensor unit: min. IP 66 |  |  |  |
|  | Components: |  |  |  |
|  | - Heading antenna unit - Processor unit (if not integrated) - Display unit - Cables to sensor unit, processor unit and display unit - Power supply cable |  |  |  |
| S_VUT_1.5 | Hardware for Inland ECDIS viewer in navigation mode |  |  |  |
|  | Features: |  |  |  |
|  | - Compliant to CCNR Inland ECDIS standard 2.0 - CCNR certified for navigation mode |  |  |  |
|  | Components: |  |  |  |
|  | - Certified PC HW - Onboard mounting kit for PC - TFT display min. 19”, certified for Navigation mode - Onboard mounting kit for TFT display - Interface for connection of digital heading device (S_VUT_1.4) - Interface for connection of Inland AIS unit (S_VUT_1.1) - Interface for connection of digital radar (S_VUT_1.3) - Display switch for change between pure radar and Inland ECDIS navigation mode view - Trackball - Operating system suitable for Inland ECDIS viewer software (navigation mode) defined under S_VUT_1.6 |  |  |  |
| S_VUT_1.6 | Software for Inland ECDIS viewer in navigation mode |  |  |  |
|  | Features: |  |  |  |
|  | - Inland ECDIS Viewer Application for Navigation mode - CCNR approval for navigation mode - Switchable to information mode - Ready to run installed on PC (S_VUT_1.5) - Functionality according to VUT_TA_1 and VUT_TA_2 - Support of S-57 charts (.000 format) - Support of Inland AIS functionality - Software support for 3 years |  |  |  |
| S_VUT_1.7 | Laptop PCs for use on vessels |  |  |  |
|  | Features: |  |  |  |
|  | - Min. 14,1” screen non-glare - Min. resolution 1280x1024 pixel - Min. dual core CPU 2x2 GHz - Min. 2 GB DDR2 RAM - Min. 160 GB HDD - internal DVD±R DL, CD±RW - Integrated wireless LAN 802.11 b/g/n - Min 4 x USB 2.0 ports - PCMCIA or PC Express card slot - VGA or DVI out - Li-Ion battery for min. 2 hours of independent operation - Integrated track pad - Suitable for operation of Inland ECDIS viewer in information mode (S_VUT_1.6) - Operating system suitable for Inland ECDIS viewer software defined under S_VUT_1.8 |  |  |  |
|  | Accessories: |  |  |  |
|  | - AC adapter with power cord - DC adapter (24V input voltage) with power cord (or AC-DC converter) - USB Trackball - Carrying case - Software support for 3 years |  |  |  |
| S_VUT_1.8 | Software for Inland ECDIS viewer in information mode |  |  |  |
|  | Features |  |  |  |
|  | - Inland ECDIS Viewer Application for information mode - Support of heads-up display - Install CD - Functionality according to VUT_TA_1 - Support of S-57 charts (.000 format) - Support of Inland AIS functionality - Software pre-installed on the Laptop PCs (S_VUT_1.7) |  |  |  |
| S_VUT_1.9 | IALA MF DGPS receiver for vessel installation |  |  |  |
|  | Features: |  |  |  |
|  | - Min. GPS L1 band receiver - Min. 12 channels GPS and 2 channels for IALA beacon - Supply voltage range min.12-30 VDC - RTCM SC 104 V2.3 output - Standard NMEA-0183 V3.01 output - Min. 2 serial (RS 232) ports - Position accuracy autonomous (CEP) <= 2m - Position accuracy local DGPS (CEP) <= 0,5m |  |  |  |
|  | Components: |  |  |  |
|  | - IALA MF DGPS receiver unit - IALA MF DGPS receiver unit mounting accessories - Power supply cable for IALA MF DGPS receiver unit, 10 m length (fused) - External power switch for IALA MF DGPS receiver unit - DC-DC converter (optional) |  |  |  |
|  | Accessories: |  |  |  |
|  | - MF Omni directional antenna and GPS antenna (or integrated) - Mounting equipment for antennas - Antenna cables, 20 m length (with suitable attenuation) - One sensor interface cable for Inland AIS transponder unit, 10 m length - Operation manual - Installation manual - One page short user manual (Serbian language) |  |  |  |
|  | Environmental conditions: |  |  |  |
|  | - operating temperature -15°C to +55°C |  |  |  |

Delivery Schedules

| Lot/ Tranche | Delivery on site*) |
| --- | --- |
| Lot 1 |  |
| Tranche 1 | 90 days |
| Tranche 2 | 180 days |
| Lot 2 |  |
| Tranche 1 | 90 days |
| Tranche 2 | 180 days |
| Tranche 3 | 330 days |

*) after issuing of Commencement Order

| Item # Lot 1 | Item | Amount to be delivered in Tranche |  |  |
| --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 |
| S_H_1.1 | Hull DB Server |  | 1 |  |
| S_H_1.2 | Minimum Hull DB Server |  | 1 |  |
| S_H_1.3 | UPS |  | 1 |  |
| S_H_1.4 | KVM TFT unit |  | 1 |  |
| S_H_1.5 | Network Switch |  | 1 |  |
| S_H_1.6 | Firewall |  | 1 |  |
| S_H_1.7 | 19” rack |  | 1 |  |
| S_H_1.8 | Hull DB work stations |  | 2 |  |
| S_H_1.9 | 22” TFT LCD screens |  | 2 |  |
| S_H_1.10 | Laptop |  | 1 |  |
| S_ENC_1.1 | Mobile work station | 1 |  |  |
| S_ENC_1.2 | Workstation | 2 |  |  |
| S_ENC_1.3 | Monitor | 4 |  |  |
| S_ENC_3.1 | Operator desktop PC for multi-beam Echosounder | 1 |  |  |
| S_ENC_3.2 | Operator notebook PC for multi-beam Echosounder | 1 |  |  |
| S_ENC_3.3 | Auxiliary PC components for multi-beam Echosounder | 1 |  |  |
| S_D4D_1.2 | Workstation PC | 1 |  |  |
| S_D4D_1.3 | Monitor TFT 24 inch | 1 |  |  |
| S_D4D_1.4 | Mobile work station | 1 |  |  |
| S_LMS_1.1 | Local server |  | 2 |  |
| S_C_1.1 | Blade Server Equipment | 1 |  |  |
| S_C_1.2 | High performance work stations – LUWS Type 3 | 6 |  |  |
| S_C_1.3 | 24” TFT LCD screens | 15 |  |  |
| S_C_1.4 | A3 network colour laser copier/scanner/printer | 1 |  |  |
| S_C_1.5 | Uninterrupted power supply (UPS) | 6 |  |  |
| S_C_1.6 | Presentation Displays | 6 |  |  |
| S_C_1.7 | Data projector | 2 |  |  |
| S_C_1.8 | Mobile Data projector | 2 |  |  |
| S_C_1.9 | Test Laptop | 4 |  |  |
| S_C_1.10 | Ultra portable Laptop | 4 |  |  |
| S_C_1.11 | External HDD | 5 |  |  |
| S_C_1.12 | External DVD/CD Writer | 1 |  |  |
| S_C_1.13 | External Blue-Ray Writer | 2 |  |  |
| S_C_1.14 | RS 422 to RS 232 converter | 10 |  |  |
| S_C_1.15 | RS 422 to USB converter | 10 |  |  |
| S_C_1.16 | USB to RS 232 converter | 10 |  |  |
| S_C_1.17 | Training Laptop | 6 |  |  |
| S_C_1.18 | Office Telephone | 6 |  |  |
| S_C_1.19 | Office Furniture | 1 |  |  |
| S_C_1.20 | Training Centre Furniture | 1 |  |  |
| S_LWS_1.1 | Office PCs – LUWS Type 1 |  | 34 |  |
| S_LWS_1.2 | TFT LCD screens |  | 68 |  |
| S_LWS_1.3 | A4 colour laser printers |  | 32 |  |
| S_LWS_1.4 | Uninterrupted power supply (UPS) |  | 34 |  |
| S_LWS_1.5 | Mobile Land User Work Stations – LUWS type 2 |  | 7 |  |

| Item # Lot 2 | Item | Amount to be delivered in Tranche |  |  |
| --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 |
| S_TT_1.1 | Standard AIS Base Station type 1 | 7 |  |  |
| S_TT_1.2 | Standard AIS Base Station type 2 | 3 |  |  |
| S_TT_1.3 | Standard AIS Base Station type 3 |  | 9 |  |
| S_TT_1.4 | Directed VHF antennas | 30 |  |  |
| S_TT_1.5 | Omni VHF antennas | 3 |  |  |
| S_TT_1.6 | Two-way antenna coupler | 15 |  |  |
| S_TT_1.7 | VHF cable | 1 |  |  |
| S_TT_1.8 | GPS signal splitter | 15 |  |  |
| S_TT_1.9 | AIS DGPS data generation module | 8 |  |  |
| S_ENC_2.1 | Digital Ortho-photo images of entire Danube | 1 |  |  |
| S_D4D_1.1 | Handheld GPS receivers | 2 |  |  |
| S_LWS_1.6 | VHF radio station |  | 14 |  |
| S_VUT_1.1 | Inland AIS mobile station | 20 | 54 | 200 |
| S_VUT_1.2 | Portable Inland AIS mobile station | 10 |  |  |
| S_VUT_1.3 | Digital Radar System | 5 |  |  |
| S_VUT_1.4 | Digital Heading device | 5 |  |  |
| S_VUT_1.5 | Hardware for Inland ECDIS viewer in navigation mode | 6 |  |  |
| S_VUT_1.6 | Software for Inland ECDIS viewer in navigation mode | 5 |  |  |
| S_VUT_1.7 | Laptop PCs for use on vessels | 15 | 53 | 70 |
| S_VUT_1.8 | Software for Inland ECDIS viewer in information mode | 15 | 44 | 70 |
| S_VUT_1.9 | IALA MF DGPS receiver for vessel installation | 6 |  |  |